HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY AND EDUCATION

FACULTY OF ELECTRICAL AND ELECTRONICS ENGINEERING

FINAL COURSE PROJECT REPORT

COURSE: IOT FOUNDATIONS AND APPLICATIONS

PROJECT 03: SMART ENERGY MONITORING AND INTELLIGENT LOAD MANAGEMENT

Team members:

Dang Dinh Manh – 24119055

……………………… – …………

……………………… – …………

……………………… – …………

Supervisor: ………………………

Ho Chi Minh City, September 2026

# TABLE OF CONTENTS

(Open in Word, right-click the table of contents, choose Update Field → Update entire table to generate page numbers.)

# 1. INTRODUCTION AND PROBLEM STATEMENT

Household and small laboratory electrical demand frequently exceeds a safe threshold for short intervals when several appliances run at the same time. The consequences are nuisance breaker trips, wasted energy and shortened wiring life. Users, however, rarely know how much power they are drawing at a given moment, or which device should be disconnected before the limit is crossed.

This project builds a complete IoT system that addresses the problem at a safe model scale: it measures the current and power of two 12 V DC loads in real time, stores the history in a time-series database, displays it on a web interface, automatically sheds the low-priority load when demand exceeds a peak limit, and restores it only when conditions have been stably safe.

Specific objectives:

- Measure current and power with a quantified error; raw ADC counts are not accepted as a final measurement.

- Transmit data over MQTT with a structured topic hierarchy and detect device online/offline state through the Last Will and Testament mechanism.

- Store time-series data and expose REST APIs for history, configuration and statistics.

- Manage loads by priority, with an anti-chattering mechanism, and keep operating when the network is lost.

- Provide a real-time dashboard that clearly separates the requested state from the state the device has actually confirmed.

- Add an advanced component: peak demand detection and a daily energy budget.

Scope: the entire power circuit operates at 12 VDC and no part touches the mains supply, in accordance with the safety requirement of the assignment.

# 2. REQUIREMENTS AND THEORETICAL BACKGROUND

## 2.1. Requirement mapping

| Requirement | Implementation in this project |
| --- | --- |
| Isolated, low-voltage current measurement | ACS712-05B Hall-effect sensor, 12 V DC loads |
| At least two controllable loads | 12 V fan (high priority) and 12 V LED lamp (low priority) |
| MQTT with structured topics, QoS, availability | Six topics under smartenergy/node9988, retained LWT |
| Time-series database and REST API | SQLite or PostgreSQL, five tables, twelve endpoints |
| Dashboard with full metrics and alarms | Real-time web interface, charts, event log |
| Priority-based load management, anti-chattering | State machine with hysteresis, minimum time and power prediction |
| Requested state distinct from acknowledged state | cmd_id and ack message pair |
| Advanced component | Peak demand detection and daily energy budget |
| Safe behaviour during network loss | Local safe mode, RTC timekeeping, store-and-forward buffer |

Table 2.1. Mapping between assignment requirements and the implemented solution

## 2.2. Hall-effect current sensing

The ACS712 routes the load current through an integrated copper conduction path. The magnetic field produced is proportional to the current, is sensed by a Hall element and converted into an output voltage. The 5 A variant has a sensitivity of 185 mV/A and an output of half the supply voltage at zero current, that is 2.5 V on a 5 V rail. The conduction path is galvanically isolated from the signal circuit with a nominal withstand voltage of 2.1 kV, which is why this device satisfies the isolated-measurement requirement of the assignment.

I [A] = (V_out - V_zero) / 0.185

V_out : voltage at the sensor OUT pin (V)

V_zero: output voltage at zero current, obtained by calibration

Because the load is DC, the mean of many samples reflects the true load current and no RMS computation is required. Conversely, applying an RMS formula to a noisy DC signal adds the noise in quadrature and biases the result upwards; this was an actual defect found and corrected during development, described in chapter 12.

## 2.3. Power and energy

P [W] = U x I            (U = 12 V, treated as constant)

E [Wh] = sum of P x dt / 3600

Cost = E/1000 x tariff [currency per kWh]

The backend recomputes energy by trapezoidal integration over the telemetry series in order to cross-check the accumulator kept on the device. Gaps longer than ten seconds are excluded from the integral so that a loss of connectivity does not produce a fabricated value.

## 2.4. Hysteresis control and anti-chattering theory

A controller with a single threshold oscillates continuously whenever the measured quantity sits near that threshold. The classical remedy is a hysteresis band: shed the load when P exceeds P_peak but restore it only when P falls below P_peak minus H. For this system hysteresis alone is still insufficient, because restoring the load itself raises the demand and may immediately exceed the limit again. The project therefore applies three simultaneous conditions: a hysteresis band in power, a minimum dwell time, and a prediction step for the demand after restoration.

## 2.5. The MQTT protocol

MQTT is a publish/subscribe protocol over TCP, well suited to embedded devices because of its small packets and long-lived connections. It defines three quality-of-service levels: QoS 0 delivers at most once, QoS 1 at least once and QoS 2 exactly once. The Last Will and Testament mechanism lets the broker publish a predefined message when it loses contact with a device, so failures are detected without polling. The retained flag makes a newly subscribed client receive the most recent message on a topic immediately.

# 3. SYSTEM ARCHITECTURE

[12V adapter]-[2A fuse]-+-[LM2596 -> 5V]-> ESP32-S3, ACS712,

|                   relays, OLED, DS3231

+-[ACS712]-+-[Relay 1]- 12V fan  (HIGH priority)

+-[Relay 2]- 12V LED  (LOW priority)

ESP32-S3: 4 sample blocks/s, local control loop every 2 s,

RTC timekeeping, 600-record store-and-forward buffer

| MQTT 1883   smartenergy/node9988/{telemetry,status,state,event,cmd,ack}

v

MQTT broker --WSS 8884--> Dashboard (Vercel): live data, commands,

latency measurement

| MQTT 1883

v

FastAPI backend (Render, 24/7) -- SQLite / PostgreSQL (Neon)

-- REST /api/* + data assistant /api/chat

Figure 3.1. Overall system architecture

The system has four layers. The device layer (ESP32-S3) performs measurement and real-time control; every switching decision is taken here, so protection does not depend on the network. The communication layer is the MQTT broker, which mediates between device, backend and interface. The data layer is the FastAPI backend: it subscribes to every topic, writes to the database and exposes REST APIs. The application layer is the web dashboard, which receives live values directly from the broker over WebSocket Secure and fetches history from the REST API.

One design point deserves emphasis. The dashboard is deployed on Vercel, a platform that serves static files and serverless functions whose lifetime is a few seconds per request. Such a platform cannot hold a persistent MQTT connection to ingest telemetry, so the ingest process must run on an always-on service (Render, or a local computer). Separating the ingest worker from the interface is therefore a technical consequence, not an arbitrary choice.

Main data flow: the ESP32 measures and publishes telemetry every two seconds; the backend writes it into the telemetry table with two timestamps, the instant the device created the packet and the instant the server received it, which makes end-to-end latency measurable. The control flow runs in the opposite direction: the dashboard or the REST API publishes a command on the cmd topic, the device applies it and publishes an ack message; the backend records both the command and the acknowledgement and computes the latency between them.

# 4. HARDWARE

## 4.1. Bill of materials

| No. | Component | Key specification | Role |
| --- | --- | --- | --- |
| 1 | ESP32-S3 DevKitC-1 N16R8 | Xtensa LX7 dual-core 240 MHz, Wi-Fi, 12-bit ADC | Main controller |
| 2 | ACS712-05B | ±5 A, 185 mV/A, 2.1 kV isolation | Load current sensing |
| 3 | 2-channel 5 V relay module | Opto-isolated, active low, 10 A/30 VDC contacts | Load switching |
| 4 | LM2596 buck converter | 4–35 V in, 5.0 V out, 3 A | Supply for the control circuit |
| 5 | DS3231 | I2C RTC, ±2 ppm, battery backup | Timekeeping without network |
| 6 | SSD1306 OLED | 0.96 inch, 128×64, I2C | Local display |
| 7 | 12 V DC fan | about 0.2–0.4 A | Load 1, high priority |
| 8 | 12 V LED bar | about 0.1–0.3 A | Load 2, low priority |
| 9 | 12 V – 2 A adapter |  | Main supply |
| 10 | 2 A fuse, 1N4007 diode, 10 kΩ/20 kΩ resistors |  | Protection and ADC divider |

Table 4.1. Bill of materials

## 4.2. Pin assignment

| Component | Pin | ESP32-S3 | Function |
| --- | --- | --- | --- |
| ACS712 | OUT (through 10 kΩ/20 kΩ divider) | GPIO 1 (ADC1_CH0) | Analogue current signal |
| Relay module | IN1 | GPIO 12 | Load 1 – 12 V fan |
| Relay module | IN2 | GPIO 13 | Load 2 – 12 V LED |
| DS3231 (0x68) | SDA / SCL | GPIO 8 / GPIO 9 | Real-time clock |
| SSD1306 (0x3C) | SDA / SCL | GPIO 8 / GPIO 9 | Display, shared I2C bus |
| Buzzer via S8050 | Base through 1 kΩ | GPIO 14 | Over-limit alarm |
| LM2596 | OUT+ / OUT− | 5V / GND | 5 V supply |

Table 4.2. Pin connection table

## 4.3. Protection design and engineering decisions

- A 10 kΩ/20 kΩ divider at the ACS712 output: at 5 A the sensor output reaches 3.43 V, beyond the safe ADC range of the ESP32-S3. The ratio of 0.667 maps the 0–5 V range onto 0–3.33 V.

- A 2 A fuse on the +12 V line, placed ahead of the sensor, protects the entire power circuit.

- A 1N4007 diode in reverse parallel with the fan suppresses the back-EMF produced when the inductive load is switched off.

- Relay contacts rated 10 A/30 VDC, many times the actual load current of under 1 A, provide a wide safety margin.

- Removing the JD-VCC jumper on the relay module and supplying VCC from 3.3 V ensures the ESP32 logic high fully turns the optocoupler off.

- The ESP32-S3 was chosen over the classic ESP32 because GPIO 12 on the latter is the MTDI strapping pin; a high level at boot selects the wrong flash voltage and prevents start-up.

- The LM2596 is fed upstream of the sensor so that the control circuit current is not counted in the load measurement.

# 5. PROTOCOL AND NETWORKING

## 5.1. Topic hierarchy and quality of service

| Topic | Direction | QoS / retain | Content |
| --- | --- | --- | --- |
| smartenergy/node9988/telemetry | Device → system | QoS 0 | Measurements, every 2 s |
| smartenergy/node9988/status | Device → system | QoS 1, retained, LWT | online / offline |
| smartenergy/node9988/state | Device → system | retained | Configuration and load state |
| smartenergy/node9988/event | Device → system | QoS 0 | Shedding, restoration, alarms |
| smartenergy/node9988/cmd | Web/backend → device | QoS 1 | Commands carrying cmd_id |
| smartenergy/node9988/ack | Device → system | QoS 0 | Command acknowledgement and actual state |

Table 5.1. MQTT topic specification

Telemetry uses QoS 0 because each message completely supersedes the previous one; loss is detected through the monotonically increasing seq field, which also yields a packet success rate without the handshake overhead of QoS 1. Commands are subscribed at QoS 1. The PubSubClient library on the ESP32 only publishes at QoS 0, so command reliability is guaranteed at application level by the cmd_id/ack pair with a five second timeout; this is a known limitation stated in chapter 13.

## 5.2. Message format

{"dev":"node9988","ts":1789550319106,"seq":42,

"current_a":0.352,"power_w":4.224,"energy_today_wh":0.0931,

"energy_total_wh":1.204,"peak_today_w":4.301,

"fan":true,"led":true,"led_shed":"NONE","auto_mode":true,"safe_mode":false,

"peak_limit_w":3.5,"hysteresis_w":0.5,"budget_wh":0,"led_est_w":1.8,

"alarm_over":false,"alarm_budget":false,"sensor_mv":2565.1,"zero_mv":2500.0,

"diff_mv":65.1,"rssi":-55,"uptime_s":84,"time_src":"ntp","buffered_count":0}

The ts field is Unix time in milliseconds, taken from NTP or from the DS3231. Every message carries the device identifier, so the system scales to multiple nodes without changing the structure.

## 5.3. Availability detection

On connection the device registers an "offline" will on the status topic with the retained flag, then publishes "online" itself. If the device loses power or drops off the network, the broker publishes "offline" once the fifteen second keepalive expires, so the dashboard and the backend learn exactly when the device stopped working. The dashboard additionally tracks data freshness: if no new packet arrives within six seconds, that is three telemetry periods, it displays a stale-data warning.

# 6. BACKEND AND DATA

## 6.1. Database schema

| Table | Main columns | Purpose |
| --- | --- | --- |
| telemetry | ts_device, ts_server, seq, current_a, power_w, energy_today_wh, peak_today_w, fan, led, led_shed, auto_mode, safe_mode, peak_limit_w, alarm_over, sensor_mv, rssi | Measurement time series |
| events | ts_device, ts_server, type, detail, power_w, limit_w | Shedding, restoration, alarms |
| availability | ts_server, status | Online/offline history |
| commands | cmd_id, source, payload, ts_client, ts_seen_server, ts_ack_server, ack_ok, latency_ms | Command log and acknowledgement latency |
| settings | key, value | Electricity tariff |

Table 6.1. Database schema

The backend supports SQLite (local development) and PostgreSQL (the Neon service for 24/7 deployment) simultaneously. The data access layer converts the syntax between the two engines, so a single code base runs in both environments. Data older than the configured retention period is purged every six hours.

## 6.2. REST endpoints

| Endpoint | Function |
| --- | --- |
| GET /api/status | Latest telemetry, availability, data freshness, energy and cost for the day |
| GET /api/history?minutes=&bucket_s= | Aggregated time series, capped at 300 points |
| GET /api/energy/daily?days= | Energy, peak and cost per day |
| GET /api/events ¦ /api/availability ¦ /api/commands | Logs |
| POST /api/command | Send a command, wait for acknowledgement, return requested and confirmed state |
| POST /api/config | Peak limit, hysteresis, budget, automatic mode |
| GET /api/stats?minutes= | Statistics for the experimental chapter |
| POST /api/chat | Data assistant |
| GET /api/export/{table}.csv | Data export for the report |

Table 6.2. REST API list

The /api/stats endpoint aggregates, over a time window: peak power, mean power, energy, total time above the limit, load switching count, packet success rate derived from gaps in seq, and both device-to-server and command-to-acknowledgement latency as mean, 95th percentile and maximum. This is exactly the data set used in the experimental chapter.

# 7. APPLICATION AND USER INTERFACE

The dashboard is a single web page. It receives live values directly from the broker over WebSocket Secure and fetches history from the REST API, so the interface keeps showing real-time data even when the backend is temporarily unavailable; only the history section is lost.

- Six metric cards: instantaneous power with its limit, current, energy for the day with a budget progress bar, estimated cost, peak power for the day, and load management mode.

- A real-time chart with a selectable window of 1, 5, 15 or 60 minutes and three series: power, peak limit and current; power on the left axis in watts, current on the right axis in amperes.

- A load control panel that separates the requested state from the state confirmed by the device, with acknowledgement latency and a timeout warning.

- A configuration panel: automatic mode, peak limit, hysteresis, daily budget, zero-point calibration, energy reset and electricity tariff.

- An alarm and event log that merges live messages with history fetched from the backend.

- A history panel: power charts over 1 hour, 6 hours, 24 hours and 7 days, plus a bar chart of daily energy for the last seven days, refreshed automatically every thirty seconds.

- An experiment panel: latency in both directions, packet success rate, unacknowledged command count, a statistics button and CSV export.

- A data assistant and an MQTT log window for troubleshooting during the live demonstration.

The local OLED shows the time from the DS3231, power, current, energy for the day, the state of both loads, Wi-Fi and MQTT icons and the number of buffered records. This demonstrates during the presentation that the device operates independently of the network.

## 7.1. Data assistant

The assistant runs entirely inside the backend. A question is normalised by stripping Vietnamese diacritics, classified into an intent by regular expressions, and then answered by querying the database directly and formatting the real figures. Because no free text is generated, there is no possibility of fabricated numbers, unlike an approach based on a large language model.

For control phrasings the system does not execute anything by itself; it returns an action descriptor, the interface shows a confirmation button, and only a user click sends the command. This is a mandatory safety constraint when natural language is allowed to operate relays.

## 7.2. Backend ingest code

The backend subscribes to all six topics. For telemetry messages the noteworthy detail is the handling of buffered packets: a packet carrying the "buffered" flag uses the device timestamp as its storage timestamp, so data forwarded after a network outage lands at the correct position on the history chart instead of piling up at the moment of reception.

def on_message(client, userdata, msg):

ts = now_ms()

kind = msg.topic.rsplit("/", 1)[-1]

raw = msg.payload.decode("utf-8", "replace")

try:

if kind == "status":

status = raw.strip().lower()

if status != live["status"]:

exec_("INSERT INTO availability(ts_server,status) VALUES(?,?)", (ts, status))

live.update(status=status, status_ts=ts)

return

data = json.loads(raw)

if kind == "telemetry":

vals = [int(v) if isinstance(v, bool) else v for v in (data.get(c) for c in TEL_COLS)]

# Gói gửi bù sau khi mất mạng: lấy mốc thời gian của thiết bị làm ts_server

ts_row = int(data["ts"]) if data.get("buffered") and data.get("ts") else ts

exec_(f"INSERT INTO telemetry(dev,ts_device,ts_server,seq,{','.join(TEL_COLS)}) "

f"VALUES(?,?,?,?,{','.join('?' * len(TEL_COLS))})",

(data.get("dev"), data.get("ts") or None, ts_row, data.get("seq"), *vals))

if not data.get("buffered") and live["status"] != "online":

live.update(status="online", status_ts=ts)

elif kind == "state":

live["state"] = data

elif kind == "event":

exec_("INSERT INTO events(dev,ts_device,ts_server,type,detail,power_w,limit_w) VALUES(?,?,?,?,?,?,?)",

(data.get("dev"), data.get("ts") or None, ts, data.get("type"), data.get("detail"),

data.get("power_w"), data.get("limit_w")))

elif kind == "cmd":

cidv = data.get("cmd_id") or f"anon-{ts}"

db.insert_ignore("commands", ["cmd_id", "dev", "source", "payload", "ts_client", "ts_seen_server"],

(cidv, DEVICE_ID, data.get("src", "unknown"), raw, data.get("ts"), ts), "cmd_id")

exec_("UPDATE commands SET ts_seen_server=COALESCE(ts_seen_server,?) WHERE cmd_id=?", (ts, cidv))

elif kind == "ack":

cidv = data.get("cmd_id", "")

row = q("SELECT ts_seen_server FROM commands WHERE cmd_id=?", (cidv,), one=True)

lat = ts - row["ts_seen_server"] if row and row["ts_seen_server"] else None

exec_("UPDATE commands SET ts_ack_server=?, ack_ok=?, ack_error=?, ack_state=?, latency_ms=? WHERE cmd_id=?",

(ts, int(bool(data.get("ok"))), data.get("error"), raw, lat, cidv))

live["state"] = {k: v for k, v in data.items() if k not in ("cmd_id", "cmd_ts", "ok", "error")}

if cidv in ack_waiters:

ack_waiters[cidv].set()

except Exception as e:

print(f"[MQTT] bad message on {msg.topic}: {e} | {raw[:120]}")

## 7.3. Database access layer

The DB class lets the same source run on SQLite during development and on PostgreSQL in production by translating parameter markers and conditional insert syntax:

class DB:

def __init__(self, url: str, sqlite_path: str):

self.pg = url.startswith(("postgres://", "postgresql://"))

if self.pg:

import psycopg

from psycopg.rows import dict_row

from psycopg_pool import ConnectionPool

self.pool = ConnectionPool(url, min_size=1, max_size=5, timeout=15,

kwargs={"row_factory": dict_row, "autocommit": True})

self.pool.wait(timeout=30)

print(f"[DB] PostgreSQL: {url.split('@')[-1].split('?')[0]}")

else:

self.conn = sqlite3.connect(sqlite_path, check_same_thread=False)

self.conn.row_factory = sqlite3.Row

self.lock = threading.Lock()

print(f"[DB] SQLite: {sqlite_path}")

def _sql(self, sql: str) -> str:

return sql.replace("?", "%s") if self.pg else sql

def query(self, sql, args=(), one=False):

if self.pg:

with self.pool.connection() as c:

rows = c.execute(self._sql(sql), args).fetchall()

else:

with self.lock:

rows = [dict(r) for r in self.conn.execute(sql, args).fetchall()]

return (rows[0] if rows else None) if one else rows

def execute(self, sql, args=()):

if self.pg:

with self.pool.connection() as c:

c.execute(self._sql(sql), args)

else:

with self.lock:

self.conn.execute(sql, args)

self.conn.commit()

def insert_ignore(self, table, cols, args, key):

ph = ",".join("?" * len(cols))

tail = f" ON CONFLICT ({key}) DO NOTHING" if self.pg else ""

head = "INSERT INTO" if self.pg else "INSERT OR IGNORE INTO"

self.execute(f"{head} {table}({','.join(cols)}) VALUES({ph}){tail}", args)

def upsert_setting(self, key, value):

if self.pg:

self.execute("INSERT INTO settings VALUES(?,?) ON CONFLICT (key) DO UPDATE SET value=EXCLUDED.value", (key, value))

else:

self.execute("INSERT OR REPLACE INTO settings VALUES(?,?)", (key, value))

def init_schema(self):

## 7.4. Real-time chart code

The browser keeps a WebSocket connection to the broker and appends a point as soon as a message arrives, rather than polling the server. The buffer is trimmed by time rather than by point count, so changing the display window does not distort the horizontal axis:

function rtRedraw() {

const cut = Date.now() - RT_WINDOW_MIN * 60000;

while (rtBuf.length && rtBuf[0].t < cut) rtBuf.shift();

while (rtBuf.length > RT_MAX_POINTS) rtBuf.shift();

rt.data.labels = rtBuf.map(d => timeStr(d.t));

rt.data.datasets[0].data = rtBuf.map(d => d.p);

rt.data.datasets[1].data = rtBuf.map(d => d.lim);

rt.data.datasets[2].data = rtBuf.map(d => d.i);

rt.update('none');

$('rt-n').textContent = rtBuf.length;

}

function rtPush(t, p, limit, i) {

rtBuf.push({ t, p, lim: limit, i });

if (!S.paused) rtRedraw();

}

$('rt-range').onchange = e => {

RT_WINDOW_MIN = Number(e.target.value);

rtBuf.length = 0;                                  // nạp lại từ backend cho đủ cửa sổ mới

if (S.apiOk) prefillChart(); else rtRedraw();

};

$('rt-pause').onclick = () => {

# 8. CONTROL AND SYSTEM INTELLIGENCE

## 8.1. Priority-based load shedding algorithm

Every 2 seconds, with P the 2-second mean power:

IF managed AND LED is on AND (P > P_peak continuously >= 4 s)

-> SHED the LED, state SHED_PEAK, emit event SHED

ELSE IF managed AND LED is off AND state = SHED_PEAK

AND (P < P_peak - H continuously >= 15 s)

AND (LED has been off >= 20 s)

AND (P + estimated P_LED < P_peak)

-> RESTORE the LED, emit event RESTORE

IF LED is off AND P still > P_peak continuously >= 10 s

-> ALARM_OVER_LIMIT + buzzer (the high-priority fan is never shed)

The three restoration conditions complement one another. The hysteresis band prevents oscillation caused by measurement noise around the threshold. The minimum dwell time prevents oscillation caused by rapidly varying loads. The prediction condition is the most important: the power of the LED lamp is estimated from the power step observed each time the lamp changes state, using an exponential moving average with weights 0.7 and 0.3. If the present demand plus the estimated lamp power exceeds the limit, the system does not restore, because it knows the load would have to be shed again immediately.

The fan is the high-priority load and is never shed automatically. When demand remains above the limit after the lamp has been shed, the system raises an alarm and leaves the decision to the operator instead of disconnecting an important appliance on its own.

## 8.2. Advanced component: peak detection and daily energy budget

The device continuously records the peak power of the day and publishes it in telemetry. When demand stays above the limit while the low-priority load has already been shed, the system raises a peak alarm, that is, it detects a situation in which the available shedding measure is insufficient.

The daily energy budget allows a watt-hour quota per day. At eighty per cent of the quota the system issues an early warning. Once the quota is exceeded the low-priority load is shed and is only re-enabled on the following day. This policy is fully explainable in terms of rules, which makes it convenient to defend and to verify, unlike a black-box approach.

## 8.3. Requested state versus confirmed state

Every command carries a randomly generated cmd_id. The device applies the command and then publishes an ack message containing the same cmd_id together with the actual state of both relays after application. The interface displays two separate lines, measures the latency with the browser clock and reports an error if no acknowledgement arrives within five seconds. This mechanism exposes the true nature of a distributed system: a command that has been sent is not a command that has been executed.

One case deserves attention: a user switching a load by hand while automatic mode is active. The device switches to manual mode and emits a MANUAL_OVERRIDE event, rather than letting two mechanisms fight over the same relay.

## 8.4. Measurement code

The following function runs four times per second on the ESP32. Three technical points are worth noting: averaging 64 samples to suppress noise, taking the absolute value of the deviation so that the measurement does not depend on the sensor wiring polarity, and automatically re-calibrating the zero point while both loads are off.

void measureBlock() {

unsigned long now = millis();

float dt_h = (last_block ? (now - last_block) : BLOCK_MS) / 3600000.0;

last_block = now;

last_mv = readMvAvg(SAMPLES_PER_BLOCK);

diff_mv = last_mv - zero_mv;

// TRỊ TUYỆT ĐỐI: tải một chiều, nếu lỡ đấu ngược IP+/IP- thì vẫn đo đúng độ lớn

float i = fabs(diff_mv) / ACS_MV_PER_A;

if (i < I_DEADBAND_A) i = 0;

current_a = i;

power_w = V_NOMINAL * current_a;

// TỰ HIỆU CHỈNH ĐIỂM 0: khi cả 2 tải đang TẮT, kéo zero_mv bám theo mức thực tế

// (bù trôi do đổi nguồn USB <-> LM2596, do nhiệt độ). Hằng số thời gian ~12 giây.

if (!fan_on && !led_on) {

if (!zero_track_ms) zero_track_ms = now;

if (now - zero_track_ms > 3000 && fabs(diff_mv) < 400)

zero_mv += (last_mv - zero_mv) * 0.02f;

} else {

zero_track_ms = 0;

}

energy_today_wh += power_w * dt_h;

energy_total_wh += power_w * dt_h;

sum_block_p += power_w; n_block++;

The absolute value arises from a real defect: the first version wrote "if (i < deadband) i = 0", which forced every negative value to zero. When the IP+ and IP− terminals of the sensor were connected the other way round, the output voltage fell below the 2.5 V reference, the computed current became negative and was discarded entirely; the system reported 0 A while the load was running. Chapter 12 records the full account.

The automatic zero tracking addresses reference drift when the supply changes. Measurements recorded a drift of about 30 mV between USB powering and LM2596 powering, equivalent to an error of 0.17 A if left uncompensated.

## 8.5. Load management state machine code

void controlStep() {

unsigned long now = millis();

if (learn_pending && now - led_switch_ms >= TELEMETRY_MS) {

float delta = fabs(demand_w - p_before_switch);

if (delta > 0.2) led_est_w = (led_est_w <= 0) ? delta : 0.7 * led_est_w + 0.3 * delta;

learn_pending = false;

}

bool over = demand_w > peak_limit_w;

bool safe = demand_w < peak_limit_w - hysteresis_w;

over_since = over ? (over_since ? over_since : now) : 0;

safe_since = safe ? (safe_since ? safe_since : now) : 0;

if (demand_w > peak_today_w) peak_today_w = demand_w;

long k = todayKey();

if (k && k != day_key) {

if (day_key) { energy_today_wh = 0; peak_today_w = 0; budget_warned = false; alarm_budget = false; }

day_key = k;

if (led_shed == SHED_BUDGET && (auto_mode || safe_mode)) switchLed(true, SHED_NONE);

}

bool managed = auto_mode || safe_mode;

if (budget_wh > 0) {

if (!budget_warned && energy_today_wh >= 0.8 * budget_wh) { budget_warned = true; publishEvent("BUDGET_WARN", "80% ngan sach ngay"); }

if (!alarm_budget && energy_today_wh >= budget_wh) { alarm_budget = true; publishEvent("BUDGET_EXCEEDED", "vuot ngan sach ngay"); }

}

if (managed && alarm_budget && led_on) {

switchLed(false, SHED_BUDGET);

publishEvent("SHED", "LED - vuot ngan sach ngay");

}

if (managed && led_on && over_since && now - over_since >= T_OVER_MS) {

switchLed(false, SHED_PEAK);

publishEvent("SHED", "LED - vuot nguong dinh");

} else if (managed && !led_on && led_shed == SHED_PEAK && !alarm_budget &&

safe_since && now - safe_since >= T_SAFE_MS &&

now - led_switch_ms >= MIN_OFF_MS &&

demand_w + led_est_w < peak_limit_w) {

switchLed(true, SHED_NONE);

publishEvent("RESTORE", "LED - dieu kien an toan on dinh");

}

bool should_alarm = over_since && now - over_since >= T_ALARM_MS && !led_on;

if (should_alarm && !alarm_over) { alarm_over = true; publishEvent("ALARM_OVER_LIMIT", "van vuot nguong sau khi sa thai"); }

if (!over && alarm_over) { alarm_over = false; publishEvent("ALARM_CLEAR", "cong suat da ve duoi nguong"); }

buzzer(alarm_over);

## 8.6. Command acknowledgement code

The device receives a command, applies it, then publishes an acknowledgement carrying the same command identifier and the actual state of both relays:

void onMqtt(char* topic, byte* payload, unsigned int len) {

StaticJsonDocument<512> in;

if (deserializeJson(in, (const char*)payload, len)) return;

char cmd_id[40] = "";

strlcpy(cmd_id, in["cmd_id"] | "", sizeof(cmd_id));

uint64_t t_cmd = in["ts"] | (uint64_t)0;

bool ok = true; String err = "";

if (in["fan"].is<bool>() || in["led"].is<bool>()) {

if (auto_mode) { auto_mode = false; publishEvent("MANUAL_OVERRIDE", "lenh tay -> chuyen MANUAL"); }

if (in["fan"].is<bool>()) { fan_on = in["fan"]; setRelay(PIN_RELAY_FAN, fan_on); }

if (in["led"].is<bool>()) switchLed(in["led"].as<bool>(), SHED_NONE);

}

if (in["auto_mode"].is<bool>()) auto_mode = in["auto_mode"];

if (!in["peak_limit_w"].isNull()) {

float v = in["peak_limit_w"]; if (v >= 0.5 && v <= 60) peak_limit_w = v; else { ok = false; err += "peak_limit_w ngoai [0.5,60]; "; }

}

if (!in["hysteresis_w"].isNull()) {

float v = in["hysteresis_w"]; if (v >= 0.1 && v <= 10) hysteresis_w = v; else { ok = false; err += "hysteresis_w ngoai [0.1,10]; "; }

}

if (!in["budget_wh"].isNull()) {

float v = in["budget_wh"]; if (v >= 0 && v <= 10000) { budget_wh = v; alarm_budget = false; budget_warned = false; }

else { ok = false; err += "budget_wh ngoai [0,10000]; "; }

}

if (in["reset_energy"] == true) { energy_today_wh = 0; peak_today_w = 0; alarm_budget = false; budget_warned = false; }

if (in["calibrate"] == true) {

if (fan_on || led_on) { ok = false; err += "tat ca 2 tai truoc khi calib; "; } else calibrateZero();

}

prefs.putFloat("limit", peak_limit_w); prefs.putFloat("hyst", hysteresis_w);

prefs.putFloat("budget", budget_wh);   prefs.putBool("auto", auto_mode);

StaticJsonDocument<512> ack;

ack["cmd_id"] = cmd_id; ack["cmd_ts"] = t_cmd; ack["ok"] = ok;

if (!ok) ack["error"] = err;

fillState(ack);

publishJson(T_ACK, ack);

publishState();

Serial.printf("[CMD] id=%s ok=%d %s\n", cmd_id, ok, err.c_str());

# 9. SECURITY AND RELIABILITY

## 9.1. Behaviour during network loss

- Wi-Fi and MQTT reconnection never blocks the main loop; measurement and control keep running on their two-second cycle while reconnection is attempted.

- If the broker is unreachable for more than thirty seconds the device enters safe mode: local load management is activated even if the device was in manual mode.

- The DS3231 keeps time without Wi-Fi, so records created during an outage carry correct timestamps; when the network returns, NTP time is written back to the RTC every ten minutes.

- A 600-record circular buffer, about twenty minutes, stores data during an outage and forwards it at five packets per 200 ms on reconnection. Forwarded packets carry the buffered flag and the backend uses the device timestamp, so the history chart is not distorted.

- Reconnection emits a RECOVERED event carrying the outage duration and the number of forwarded packets, which is used as evidence in the experiments.

- Critical configuration is stored in flash, so thresholds and mode survive a power cycle.

## 9.2. Security

| Threat | Impact | Current mitigation | Planned improvement |
| --- | --- | --- | --- |
| A stranger publishes on the cmd topic | Unauthorised load switching | Hard-to-guess topic name; every command is logged with its source | Authenticated broker with per-topic ACL |
| Telemetry eavesdropping | Usage patterns disclosed | No personal data in the messages | MQTT over TLS on port 8883 |
| Arbitrary REST calls | Remote configuration changes | X-API-Key header required on every POST and PUT | Per-user authentication, separate view and control rights |
| Wi-Fi password exposed in source | Unauthorised access to the local network | Moved to secrets.h and excluded from Git | Provisioning through a first-boot access point |
| The assistant misreads an intent | Unintended switching | Manual confirmation required before any command is sent | Per-user audit log |

Table 9.1. Risk analysis and mitigations

The main security limitation is the public broker without authentication. This was a conscious choice so that the system works under the network conditions available at the university, and the source is already prepared for a switch to an authenticated broker through configuration only.

# 10. EXPERIMENTAL METHOD

| ID | Experiment | Procedure | Quantities collected |
| --- | --- | --- | --- |
| E1 | Measurement error | A reference ammeter in series; four cases: no load, fan, lamp, both; 30 samples each | Absolute and percentage error, standard deviation |
| E2 | Device to dashboard latency | Compare the packet timestamp with the display instant over 100 consecutive packets | Mean, 95th percentile, maximum |
| E3 | Command to acknowledgement latency | Thirty on/off operations, read from the interface and the commands table | Mean, 95th percentile, timeouts |
| E4 | Managed versus unmanaged operation | Two ten-minute runs with identical loads and limit, automatic mode off then on | Peak power, energy, time above limit, switching count |
| E5 | Anti-chattering | Set the limit close to the combined load; compare the full configuration against one without hysteresis and minimum time | Switching operations per ten minutes |
| E6 | Network loss and recovery | Disconnect the router for sixty seconds while above the limit | Offline detection instant, local behaviour, forwarded packets, residual data gap |
| E7 | Daily energy budget | Set a small quota and observe the warning and shedding behaviour | Time of the 80 % warning and of the quota breach |
| E8 | RTC drift | Compare DS3231 time against NTP after 24 hours | Seconds of drift per day |

Table 10.1. Experiment matrix

Instrumentation: the interface displays latency and packet success rate directly; the /api/stats endpoint aggregates them over a window; CSV export allows the raw data to be reprocessed in a spreadsheet. Every figure in the results chapter comes from these three sources and none is entered by hand.

# 11. RESULTS AND DISCUSSION

The tables below are completed after running the experiments on the physical prototype.

## 11.1. Measurement error (E1)

| Case | Reference (A) | System (A) | Error (A) | Error (%) | Std. dev. (A) |
| --- | --- | --- | --- | --- | --- |
| No load |  |  |  |  |  |
| Fan only |  |  |  |  |  |
| LED lamp only |  |  |  |  |  |
| Both loads |  |  |  |  |  |

Table 11.1. Measurement error results

## 11.2. Latency (E2, E3)

| Metric | Samples | Mean (ms) | 95th percentile (ms) | Maximum (ms) |
| --- | --- | --- | --- | --- |
| Device → backend |  |  |  |  |
| Device → dashboard |  |  |  |  |
| Command → acknowledgement |  |  |  |  |

Table 11.2. Latency results

## 11.3. Managed versus unmanaged operation (E4)

| Metric over 10 minutes | Unmanaged | Managed | Difference |
| --- | --- | --- | --- |
| Peak power (W) |  |  |  |
| Energy consumed (Wh) |  |  |  |
| Time above limit (s) |  |  |  |
| Load switching operations |  |  |  |
| Packet success rate (%) |  |  |  |

Table 11.3. Effectiveness of load management

## 11.4. Anti-chattering (E5) and network loss (E6)

| Configuration | Switching operations / 10 min | Comment |
| --- | --- | --- |
| All three conditions (H = 0.5 W, 15 s, 20 s, with prediction) |  |  |
| Single threshold, no hysteresis, no minimum time |  |  |

Table 11.4. Effectiveness of the anti-chattering mechanism

| Observation (E6) | Result |
| --- | --- |
| Offline detected after disconnection (s) |  |
| Local shedding behaviour during the outage |  |
| Records forwarded after recovery |  |
| Residual gap on the chart (s) |  |
| Time from reconnection to continuous data (s) |  |

Table 11.5. Network loss experiment results

## 11.5. Discussion

The discussion should answer four questions: whether the measurement error is small enough relative to the control threshold; whether the end-to-end latency is compatible with the two-second control cycle; by what percentage load management reduces the time spent above the limit compared with unmanaged operation; and how the system behaves when the network is lost. If the measurement error exceeds roughly one tenth of the threshold, the voltage divider, the quality of the 5 V rail and the zero-point calibration should be reviewed.

# 12. DEVELOPMENT PROCESS AND DEFECTS RESOLVED

The system went through several rounds of debugging. This chapter records the defects with technical value, because they illustrate the characteristic traps of a multi-layer IoT system: a fault rarely sits in one place, it hides at the boundary between layers.

| Defect | Symptom | Root cause | Resolution |
| --- | --- | --- | --- |
| Chart never plotted | Metric cards updated but the curve stayed flat at zero | chart.data.datasets.data written instead of datasets[0].data; the exception was swallowed by a try/catch block | Fixed the array index and moved to a time-based buffer |
| Current always zero | Loads were running but power read 0 W | A clamp forced negative values to zero, so a reversed IP+/IP− connection discarded the entire measurement | Take the absolute value of the voltage deviation |
| Zero point drift | Sensor read 2499 mV while the stored zero was 2530 mV | The reference level changed when the supply moved from USB to the LM2596 | Automatic zero tracking while both loads are off, time constant of twelve seconds |
| Protection stopped during outage | Load shedding ceased when the broker was unreachable | The reconnection routine used a blocking loop with delay, which stalled the control loop | Non-blocking reconnection; the control loop was decoupled from network tasks |
| Fundamentally wrong current computation | Readings biased high at small loads | An RMS formula applied to a DC signal adds noise in quadrature | Use the sample mean, which is correct for a DC load |
| Device and web never met | Both reported a successful broker connection yet no data appeared | The old topic tree used an underscore while the new one used a slash | A single topic tree, with the topic name printed to the serial port for comparison |
| Web deployment returned 404 | The deployed page would not open | The interface file lived in a subdirectory while the platform looked for it at the repository root | Moved the interface file to the repository root |
| Continuous switching | The auxiliary load cycled rapidly around the threshold | A single threshold; after restoration the limit was exceeded again immediately | Three simultaneous conditions: hysteresis, minimum time, predicted power after restoration |

Table 12.1. Defects found and how they were resolved

The lesson is that in a distributed system a layer reporting success does not mean the system is working. The device reported a successful publish, the browser reported a successful connection, yet with two different topic names the data could never meet. The team therefore added observation points at every layer: the topic name printed to the serial port, the data freshness indicator on the interface, the backend status endpoint, and direct database queries.

# 13. LIMITATIONS

- The public MQTT broker has no authentication; anyone who knows the topic names can read the data and send commands.

- The supply voltage is assumed to be a constant 12 V rather than measured, so the accuracy of the power figure depends on the stability of the supply.

- The 5 A ACS712 has limited resolution for loads below about 0.3 A; an INA219 or INA226 would perform considerably better in that range.

- A single sensor measures the total current, so the power of an individual load is only an estimate derived from the step observed at switching.

- The PubSubClient library only publishes at QoS 0; command reliability relies on the application-level acknowledgement mechanism.

- The offline buffer lives in RAM and is lost on reset; it could be upgraded to LittleFS storage.

- The free tier of the hosting service suspends the instance without traffic, so a periodic keep-alive task is required and short data gaps remain possible.

# 14. CONCLUSION

The project delivers a complete IoT system for energy monitoring and intelligent load management, working end to end from the sensor to the web interface. It measures current and power with a documented calibration procedure, transmits data over MQTT with a structured topic hierarchy and availability detection, stores history in a time-series database with REST access, and provides a real-time dashboard that distinguishes requested state from confirmed state.

The most substantial technical contribution lies in control and reliability. The restoration mechanism based on three simultaneous conditions, including a prediction of the power after restoration, fully resolves the continuous switching that a single threshold cannot address. Decoupling the local control loop from network tasks, combined with a real-time clock and a store-and-forward buffer, preserves both the safety function and data integrity during a network outage.

Future work: migrate to an authenticated broker with TLS; replace the ACS712 with an INA226 to measure both voltage and current at higher resolution; extend the design to several measurement nodes with load coordination between them; and add short-term load forecasting so that shedding becomes anticipatory rather than reactive.

# 15. REFERENCES

[1] Allegro MicroSystems, "ACS712: Fully Integrated, Hall Effect-Based Linear Current Sensor IC", Datasheet, Rev. 18.

[2] Espressif Systems, "ESP32-S3 Technical Reference Manual", 2024.

[3] Espressif Systems, "ESP32-S3 Series Datasheet — ADC Characteristics and Calibration", 2024.

[4] Espressif Systems, "ESP32-S3-DevKitC-1 v1.1 User Guide", esp-dev-kits documentation.

[5] OASIS, "MQTT Version 5.0 — OASIS Standard", 2019.

[6] Analog Devices, "DS3231 Extremely Accurate I2C-Integrated RTC/TCXO/Crystal", Datasheet.

[7] Solomon Systech, "SSD1306 Advance Information — 128x64 Dot Matrix OLED/PLED Driver".

[8] S. Ramirez, "FastAPI Documentation", https://fastapi.tiangolo.com

[9] Eclipse Foundation, "Eclipse Paho MQTT Python Client Documentation".

[10] N. O'Leary, "PubSubClient — Arduino Client for MQTT", https://pubsubclient.knolleary.net

[11] B. Blanchon, "ArduinoJson Documentation", https://arduinojson.org

[12] K. Ogata, "Modern Control Engineering", 5th ed., Prentice Hall, 2010 (on-off control and hysteresis).

# 16. TEAM CONTRIBUTIONS

| No. | Full name | Student ID | Responsibilities | Contribution |
| --- | --- | --- | --- | --- |
| 1 | Dang Dinh Manh | 24119055 | Circuit design, ESP32-S3 firmware, load management algorithm |  |
| 2 |  |  | FastAPI backend, database, service deployment |  |
| 3 |  |  | Web dashboard, data assistant, user interface |  |
| 4 |  |  | Experiments, error and latency measurement, report writing |  |

Table 16.1. Task allocation

Full source code: https://github.com/JackGamerCYT/IOT-PROJECT3-

# APPENDIX A. COMPLETE ESP32-S3 FIRMWARE SOURCE

File firmware/esp32_firmware/esp32_firmware.ino. Built with the Arduino IDE, board ESP32S3 Dev Module, two libraries: PubSubClient and ArduinoJson. Wi-Fi credentials live in a separate secrets.h file and are therefore absent from the public repository.

1 | /*

2 |  * =====================================================================================

3 |  *  PROJECT 03 - SMART ENERGY MONITORING & INTELLIGENT LOAD MANAGEMENT

4 |  *  Firmware v21 - ESP32-S3 (N16R8)

5 |  *

6 |  *  v19 thêm so với v17/v18:

7 |  *   - DS3231 (I2C): giữ giờ khi mất WiFi; NTP -> RTC khi online, RTC -> hệ thống khi offline

8 |  *   - SSD1306 OLED 128x64 (I2C): hiện giờ, công suất, điện năng, trạng thái tải, mạng

9 |  *   - Bộ đệm vòng 600 bản ghi (~20 phút): mất broker thì lưu lại, có mạng gửi bù

10 |  *     (gói gửi bù có "buffered":true, backend dùng ts thiết bị làm mốc thời gian)

11 |  *

12 |  *  Thư viện cần cài (Library Manager):

13 |  *    PubSubClient · ArduinoJson · RTClib · Adafruit SSD1306 · Adafruit GFX Library

14 |  *  Không lắp module nào thì đặt USE_RTC / USE_OLED = 0, firmware vẫn chạy đầy đủ.

15 |  * =====================================================================================

16 |  */

17 |

18 | #define ARDUINOJSON_USE_LONG_LONG 1

19 |

20 | // ======================================================

21 | //  CHUA GAN DS3231 / OLED thi de nguyen 0 (chi can 2 thu vien PubSubClient + ArduinoJson).

22 | //  Khi nao gan module roi thi doi thanh 1 va cai them RTClib, Adafruit SSD1306, Adafruit GFX.

23 | // ======================================================

24 | #define USE_RTC  0

25 | #define USE_OLED 0

26 |

27 | #include <WiFi.h>

28 | #include <PubSubClient.h>

29 | #include <ArduinoJson.h>

30 | #include <Preferences.h>

31 | #include <time.h>

32 | #include <sys/time.h>

33 | #if USE_RTC || USE_OLED

34 |   #include <Wire.h>

35 | #endif

36 | #if USE_RTC

37 |   #include <RTClib.h>

38 | #endif

39 | #if USE_OLED

40 |   #include <Adafruit_GFX.h>

41 |   #include <Adafruit_SSD1306.h>

42 | #endif

43 |

44 | // -------------------------------------------------------------------------------------

45 | // 1. BÍ MẬT (tạo secrets.h cạnh file .ino, KHÔNG commit)

46 | // -------------------------------------------------------------------------------------

47 | #if __has_include("secrets.h")

48 |   #include "secrets.h"

49 | #else

50 |   #define WIFI_SSID     "YOUR_WIFI"

51 |   #define WIFI_PASSWORD "YOUR_PASSWORD"

52 |   #define MQTT_USER     ""

53 |   #define MQTT_PASS     ""

54 | #endif

55 |

56 | // -------------------------------------------------------------------------------------

57 | // 2. MQTT - cây topic: smartenergy/<device_id>/<kênh>

58 | // -------------------------------------------------------------------------------------

59 | const char* DEVICE_ID   = "node9988";

60 | const char* MQTT_BROKER = "broker.hivemq.com";

61 | const int   MQTT_PORT   = 1883;

62 |

63 | const char* T_TELEMETRY = "smartenergy/node9988/telemetry";

64 | const char* T_STATUS    = "smartenergy/node9988/status";

65 | const char* T_STATE     = "smartenergy/node9988/state";

66 | const char* T_EVENT     = "smartenergy/node9988/event";

67 | const char* T_CMD       = "smartenergy/node9988/cmd";

68 | const char* T_ACK       = "smartenergy/node9988/ack";

69 |

70 | // -------------------------------------------------------------------------------------

71 | // 3. PHẦN CỨNG

72 | // -------------------------------------------------------------------------------------

73 | const int PIN_ACS712    = 1;    // ADC1_CH0, nên qua cầu phân áp 10k/20k

74 | const int PIN_RELAY_FAN = 12;   // Tải 1 - Quạt 12V (ưu tiên CAO), relay active LOW

75 | const int PIN_RELAY_LED = 13;   // Tải 2 - LED 12V  (ưu tiên THẤP), relay active LOW

76 | const int PIN_BUZZER    = 14;   // -1 nếu không lắp

77 | const int PIN_I2C_SDA   = 8;    // DS3231 + SSD1306 dùng chung

78 | const int PIN_I2C_SCL   = 9;

79 | const uint8_t ADDR_OLED = 0x3C;

80 |

81 | const float ACS_MV_PER_A  = 185.0;

82 | const float DIVIDER_RATIO = 1.0;    // 0.6667 nếu dùng phân áp 10k (trên) / 20k (dưới)

83 | const float V_NOMINAL     = 12.0;

84 | const float I_DEADBAND_A  = 0.04;

85 |

86 | // -------------------------------------------------------------------------------------

87 | // 4. THÔNG SỐ ĐO / ĐIỀU KHIỂN

88 | // -------------------------------------------------------------------------------------

89 | const unsigned long BLOCK_MS          = 250;

90 | const int           SAMPLES_PER_BLOCK = 64;

91 | const unsigned long TELEMETRY_MS      = 2000;

92 | const unsigned long T_OVER_MS         = 4000;

93 | const unsigned long T_SAFE_MS         = 15000;

94 | const unsigned long MIN_OFF_MS        = 20000;

95 | const unsigned long T_ALARM_MS        = 10000;

96 | const unsigned long OFFLINE_SAFE_MS   = 30000;

97 | const unsigned long SAVE_MS           = 60000;

98 | const unsigned long OLED_MS           = 500;

99 | const int           BUF_SIZE          = 600;    // ~20 phút dữ liệu đệm

100 |

101 | float peak_limit_w = 3.0;

102 | float hysteresis_w = 0.5;

103 | float budget_wh    = 0.0;

104 | bool  auto_mode    = false;

105 |

106 | // -------------------------------------------------------------------------------------

107 | // 5. BIẾN TRẠNG THÁI

108 | // -------------------------------------------------------------------------------------

109 | enum ShedReason { SHED_NONE = 0, SHED_PEAK, SHED_BUDGET };

110 | const char* shedName(int r) { return r == SHED_PEAK ? "PEAK" : r == SHED_BUDGET ? "BUDGET" : "NONE"; }

111 |

112 | bool fan_on = false, led_on = false;

113 | int  led_shed = SHED_NONE;

114 | unsigned long led_switch_ms = 0;

115 |

116 | float zero_mv = 2500.0, last_mv = 0, diff_mv = 0;

117 | unsigned long zero_track_ms = 0;

118 | float current_a = 0, power_w = 0, demand_w = 0;

119 | double sum_block_p = 0; int n_block = 0;

120 | double energy_today_wh = 0, energy_total_wh = 0;

121 | float  peak_today_w = 0;

122 | long   day_key = 0;

123 |

124 | unsigned long over_since = 0, safe_since = 0;

125 | float led_est_w = 0, p_before_switch = -1;

126 | bool learn_pending = false;

127 | bool alarm_over = false, alarm_budget = false, budget_warned = false;

128 |

129 | bool safe_mode = false;

130 | unsigned long mqtt_lost_ms = 0;

131 | unsigned long last_block = 0, last_tele = 0, last_save = 0, last_mqtt_try = 0,

132 |               last_wifi_try = 0, last_oled = 0, last_flush = 0;

133 | unsigned long seq = 0;

134 |

135 | // Bộ đệm vòng khi mất kết nối

136 | struct Rec { uint32_t ts; float i, p, e; uint8_t flags; };

137 | Rec buf[BUF_SIZE];

138 | int buf_head = 0, buf_count = 0;

139 |

140 | bool have_rtc = false, have_oled = false;

141 | const char* time_src = "none";

142 |

143 | WiFiClient net;

144 | PubSubClient mqtt(net);

145 | Preferences prefs;

146 | String client_id;

147 | #if USE_RTC

148 |   RTC_DS3231 rtc;

149 | #endif

150 | #if USE_OLED

151 |   Adafruit_SSD1306 oled(128, 64, &Wire, -1);

152 | #endif

153 |

154 | // -------------------------------------------------------------------------------------

155 | uint64_t epochMs() {

156 |   struct timeval tv; gettimeofday(&tv, nullptr);

157 |   if (tv.tv_sec < 1700000000) return 0;

158 |   return (uint64_t)tv.tv_sec * 1000ULL + tv.tv_usec / 1000;

159 | }

160 | uint32_t epochS() { time_t t = time(nullptr); return t > 1700000000 ? (uint32_t)t : 0; }

161 |

162 | long todayKey() {

163 |   time_t now = time(nullptr); if (now < 1700000000) return 0;

164 |   struct tm t; localtime_r(&now, &t);

165 |   return (t.tm_year + 1900) * 10000L + (t.tm_mon + 1) * 100L + t.tm_mday;

166 | }

167 | void clockStr(char* out, size_t n) {

168 |   time_t now = time(nullptr);

169 |   if (now < 1700000000) { snprintf(out, n, "--:--:--"); return; }

170 |   struct tm t; localtime_r(&now, &t);

171 |   snprintf(out, n, "%02d:%02d:%02d", t.tm_hour, t.tm_min, t.tm_sec);

172 | }

173 |

174 | const char* mqttErr(int rc) {

175 |   switch (rc) {

176 |     case -4: return "het thoi gian cho broker tra loi";

177 |     case -3: return "mat ket noi mang";

178 |     case -2: return "KHONG KET NOI DUOC TCP (mang chan cong 1883?)";

179 |     case -1: return "broker tu choi / ngat ket noi";

180 |     case  0: return "OK";

181 |     case  1: return "broker khong ho tro phien ban MQTT";

182 |     case  2: return "client_id bi tu choi";

183 |     case  3: return "broker dang ban";

184 |     case  4: return "sai user/password";

185 |     case  5: return "khong duoc phep ket noi";

186 |     default: return "khong ro";

187 |   }

188 | }

189 |

190 | const char* wifiErr(int st) {

191 |   switch (st) {

192 |     case WL_NO_SSID_AVAIL: return "KHONG TIM THAY TEN WIFI (sai SSID?)";

193 |     case WL_CONNECT_FAILED: return "SAI MAT KHAU WIFI";

194 |     case WL_CONNECTION_LOST: return "mat song";

195 |     case WL_DISCONNECTED: return "dang ket noi...";

196 |     case WL_IDLE_STATUS: return "dang cho";

197 |     case WL_CONNECTED: return "da ket noi";

198 |     default: return "khong ro";

199 |   }

200 | }

201 |

202 | void setRelay(int pin, bool on) { digitalWrite(pin, on ? LOW : HIGH); }

203 | void buzzer(bool on) { if (PIN_BUZZER >= 0) digitalWrite(PIN_BUZZER, on ? HIGH : LOW); }

204 |

205 | void publishJson(const char* topic, JsonDocument& doc, bool retained = false) {

206 |   if (!mqtt.connected()) return;

207 |   char b[768];

208 |   size_t n = serializeJson(doc, b, sizeof(b));

209 |   mqtt.publish(topic, (const uint8_t*)b, n, retained);

210 | }

211 |

212 | void publishEvent(const char* type, const char* detail) {

213 |   Serial.printf("[EVENT] %s %s | P=%.2fW limit=%.2fW\n", type, detail, demand_w, peak_limit_w);

214 |   StaticJsonDocument<384> d;

215 |   d["dev"] = DEVICE_ID; d["ts"] = epochMs(); d["type"] = type; d["detail"] = detail;

216 |   d["power_w"] = demand_w; d["limit_w"] = peak_limit_w; d["energy_today_wh"] = energy_today_wh;

217 |   publishJson(T_EVENT, d);

218 | }

219 |

220 | void fillState(JsonDocument& d) {

221 |   d["dev"] = DEVICE_ID; d["ts"] = epochMs();

222 |   d["fan"] = fan_on; d["led"] = led_on; d["led_shed"] = shedName(led_shed);

223 |   d["auto_mode"] = auto_mode; d["safe_mode"] = safe_mode;

224 |   d["peak_limit_w"] = peak_limit_w; d["hysteresis_w"] = hysteresis_w; d["budget_wh"] = budget_wh;

225 | }

226 | void publishState() { StaticJsonDocument<384> d; fillState(d); publishJson(T_STATE, d, true); }

227 |

228 | void switchLed(bool on, int reason) {

229 |   if (on == led_on) { led_shed = on ? SHED_NONE : reason; return; }

230 |   p_before_switch = demand_w; learn_pending = true;

231 |   led_on = on; led_shed = on ? SHED_NONE : reason; led_switch_ms = millis();

232 |   setRelay(PIN_RELAY_LED, on);

233 |   publishState();

234 | }

235 |

236 | // -------------------------------------------------------------------------------------

237 | // 6. ĐỒNG HỒ THỜI GIAN THỰC

238 | // -------------------------------------------------------------------------------------

239 | void initRTC() {

240 | #if USE_RTC

241 |   if (!rtc.begin(&Wire)) { Serial.println("[RTC] khong tim thay DS3231"); return; }

242 |   have_rtc = true;

243 |   if (rtc.lostPower()) Serial.println("[RTC] mat nguon, gio chua dung - se chinh lai khi co NTP");

244 |   DateTime n = rtc.now();

245 |   if (n.unixtime() > 1700000000) {   // lấy giờ RTC làm giờ hệ thống ngay khi khởi động

246 |     struct timeval tv; tv.tv_sec = (time_t)n.unixtime(); tv.tv_usec = 0;

247 |     settimeofday(&tv, nullptr);

248 |     time_src = "rtc";

249 |     Serial.printf("[RTC] gio tu DS3231: %04d-%02d-%02d %02d:%02d:%02d\n",

250 |                   n.year(), n.month(), n.day(), n.hour(), n.minute(), n.second());

251 |   }

252 | #endif

253 | }

254 |

255 | void syncRtcFromNtp() {

256 | #if USE_RTC

257 |   if (!have_rtc) return;

258 |   uint32_t s = epochS();

259 |   if (!s) return;

260 |   DateTime cur = rtc.now();

261 |   long diff = (long)cur.unixtime() - (long)s;

262 |   if (diff > 2 || diff < -2) {                     // lệch quá 2 s mới ghi lại

263 |     rtc.adjust(DateTime(s));

264 |     Serial.println("[RTC] da dong bo DS3231 theo NTP");

265 |   }

266 | #endif

267 | }

268 |

269 | // -------------------------------------------------------------------------------------

270 | // 7. OLED

271 | // -------------------------------------------------------------------------------------

272 | void initOLED() {

273 | #if USE_OLED

274 |   if (!oled.begin(SSD1306_SWITCHCAPVCC, ADDR_OLED)) { Serial.println("[OLED] khong tim thay SSD1306"); return; }

275 |   have_oled = true;

276 |   oled.clearDisplay(); oled.setTextColor(SSD1306_WHITE); oled.setTextSize(1);

277 |   oled.setCursor(0, 24); oled.println(" SMART ENERGY v19"); oled.display();

278 | #endif

279 | }

280 |

281 | void drawOLED() {

282 | #if USE_OLED

283 |   if (!have_oled) return;

284 |   char t[16]; clockStr(t, sizeof(t));

285 |   oled.clearDisplay();

286 |

287 |   oled.setTextSize(1); oled.setCursor(0, 0); oled.print(t);

288 |   oled.setCursor(70, 0);

289 |   oled.print(WiFi.status() == WL_CONNECTED ? "W" : "-");

290 |   oled.print(mqtt.connected() ? "M" : "-");

291 |   if (buf_count) { oled.print(" B"); oled.print(buf_count); }

292 |   else { oled.print(auto_mode ? " AUT" : " MAN"); }

293 |

294 |   oled.drawFastHLine(0, 10, 128, SSD1306_WHITE);

295 |

296 |   oled.setTextSize(2); oled.setCursor(0, 15);

297 |   oled.print(demand_w, 2); oled.setTextSize(1); oled.print(" W");

298 |

299 |   oled.setCursor(0, 34);

300 |   oled.print(current_a, 3); oled.print("A  ");

301 |   oled.print(energy_today_wh, 2); oled.print("Wh");

302 |

303 |   oled.setCursor(0, 45);

304 |   oled.print("Lim "); oled.print(peak_limit_w, 1); oled.print("W");

305 |   if (alarm_over)       oled.print(" !OVER");

306 |   else if (alarm_budget) oled.print(" !BUDG");

307 |   else if (safe_mode)    oled.print(" SAFE");

308 |

309 |   oled.setCursor(0, 56);

310 |   oled.print("FAN:"); oled.print(fan_on ? "ON " : "OFF");

311 |   oled.print(" LED:"); oled.print(led_on ? "ON" : "OFF");

312 |   if (!led_on && led_shed != SHED_NONE) { oled.print("/"); oled.print(shedName(led_shed)); }

313 |   oled.display();

314 | #endif

315 | }

316 |

317 | // -------------------------------------------------------------------------------------

318 | // 8. ĐO LƯỜNG

319 | // -------------------------------------------------------------------------------------

320 | float readMvAvg(int n) {

321 |   uint32_t s = 0;

322 |   for (int i = 0; i < n; i++) s += analogReadMilliVolts(PIN_ACS712);

323 |   return (float)s / n / DIVIDER_RATIO;

324 | }

325 |

326 | void calibrateZero() {

327 |   float s = 0;

328 |   for (int i = 0; i < 20; i++) { s += readMvAvg(50); delay(10); }

329 |   zero_mv = s / 20.0;

330 |   Serial.printf("[CAL] zero = %.1f mV (ly thuyet ~2500 mV khi VCC = 5.0 V)\n", zero_mv);

331 | }

332 |

333 | void measureBlock() {

334 |   unsigned long now = millis();

335 |   float dt_h = (last_block ? (now - last_block) : BLOCK_MS) / 3600000.0;

336 |   last_block = now;

337 |

338 |   last_mv = readMvAvg(SAMPLES_PER_BLOCK);

339 |   diff_mv = last_mv - zero_mv;

340 |

341 |   // TRỊ TUYỆT ĐỐI: tải một chiều, nếu lỡ đấu ngược IP+/IP- thì vẫn đo đúng độ lớn

342 |   float i = fabs(diff_mv) / ACS_MV_PER_A;

343 |   if (i < I_DEADBAND_A) i = 0;

344 |   current_a = i;

345 |   power_w = V_NOMINAL * current_a;

346 |

347 |   // TỰ HIỆU CHỈNH ĐIỂM 0: khi cả 2 tải đang TẮT, kéo zero_mv bám theo mức thực tế

348 |   // (bù trôi do đổi nguồn USB <-> LM2596, do nhiệt độ). Hằng số thời gian ~12 giây.

349 |   if (!fan_on && !led_on) {

350 |     if (!zero_track_ms) zero_track_ms = now;

351 |     if (now - zero_track_ms > 3000 && fabs(diff_mv) < 400)

352 |       zero_mv += (last_mv - zero_mv) * 0.02f;

353 |   } else {

354 |     zero_track_ms = 0;

355 |   }

356 |

357 |   energy_today_wh += power_w * dt_h;

358 |   energy_total_wh += power_w * dt_h;

359 |   sum_block_p += power_w; n_block++;

360 | }

361 |

362 | // -------------------------------------------------------------------------------------

363 | // 9. MÁY TRẠNG THÁI QUẢN LÝ TẢI (chạy cả khi mất mạng)

364 | // -------------------------------------------------------------------------------------

365 | void controlStep() {

366 |   unsigned long now = millis();

367 |

368 |   if (learn_pending && now - led_switch_ms >= TELEMETRY_MS) {

369 |     float delta = fabs(demand_w - p_before_switch);

370 |     if (delta > 0.2) led_est_w = (led_est_w <= 0) ? delta : 0.7 * led_est_w + 0.3 * delta;

371 |     learn_pending = false;

372 |   }

373 |

374 |   bool over = demand_w > peak_limit_w;

375 |   bool safe = demand_w < peak_limit_w - hysteresis_w;

376 |   over_since = over ? (over_since ? over_since : now) : 0;

377 |   safe_since = safe ? (safe_since ? safe_since : now) : 0;

378 |   if (demand_w > peak_today_w) peak_today_w = demand_w;

379 |

380 |   long k = todayKey();

381 |   if (k && k != day_key) {

382 |     if (day_key) { energy_today_wh = 0; peak_today_w = 0; budget_warned = false; alarm_budget = false; }

383 |     day_key = k;

384 |     if (led_shed == SHED_BUDGET && (auto_mode || safe_mode)) switchLed(true, SHED_NONE);

385 |   }

386 |

387 |   bool managed = auto_mode || safe_mode;

388 |

389 |   if (budget_wh > 0) {

390 |     if (!budget_warned && energy_today_wh >= 0.8 * budget_wh) { budget_warned = true; publishEvent("BUDGET_WARN", "80% ngan sach ngay"); }

391 |     if (!alarm_budget && energy_today_wh >= budget_wh) { alarm_budget = true; publishEvent("BUDGET_EXCEEDED", "vuot ngan sach ngay"); }

392 |   }

393 |   if (managed && alarm_budget && led_on) {

394 |     switchLed(false, SHED_BUDGET);

395 |     publishEvent("SHED", "LED - vuot ngan sach ngay");

396 |   }

397 |

398 |   if (managed && led_on && over_since && now - over_since >= T_OVER_MS) {

399 |     switchLed(false, SHED_PEAK);

400 |     publishEvent("SHED", "LED - vuot nguong dinh");

401 |   } else if (managed && !led_on && led_shed == SHED_PEAK && !alarm_budget &&

402 |              safe_since && now - safe_since >= T_SAFE_MS &&

403 |              now - led_switch_ms >= MIN_OFF_MS &&

404 |              demand_w + led_est_w < peak_limit_w) {

405 |     switchLed(true, SHED_NONE);

406 |     publishEvent("RESTORE", "LED - dieu kien an toan on dinh");

407 |   }

408 |

409 |   bool should_alarm = over_since && now - over_since >= T_ALARM_MS && !led_on;

410 |   if (should_alarm && !alarm_over) { alarm_over = true; publishEvent("ALARM_OVER_LIMIT", "van vuot nguong sau khi sa thai"); }

411 |   if (!over && alarm_over) { alarm_over = false; publishEvent("ALARM_CLEAR", "cong suat da ve duoi nguong"); }

412 |   buzzer(alarm_over);

413 | }

414 |

415 | // -------------------------------------------------------------------------------------

416 | // 10. TELEMETRY + BỘ ĐỆM OFFLINE

417 | // -------------------------------------------------------------------------------------

418 | void bufferPush() {

419 |   uint32_t ts = epochS();

420 |   if (!ts) return;                       // không có mốc thời gian đáng tin thì không đệm

421 |   buf[buf_head] = { ts, current_a, demand_w, (float)energy_today_wh,

422 |                     (uint8_t)((fan_on ? 1 : 0) | (led_on ? 2 : 0) | (auto_mode ? 4 : 0) | (safe_mode ? 8 : 0)) };

423 |   buf_head = (buf_head + 1) % BUF_SIZE;

424 |   if (buf_count < BUF_SIZE) buf_count++;

425 | }

426 |

427 | void bufferFlush(int max_msgs) {

428 |   while (buf_count > 0 && max_msgs-- > 0 && mqtt.connected()) {

429 |     int idx = (buf_head - buf_count + BUF_SIZE) % BUF_SIZE;

430 |     Rec& r = buf[idx];

431 |     StaticJsonDocument<384> d;

432 |     d["dev"] = DEVICE_ID; d["ts"] = (uint64_t)r.ts * 1000ULL; d["buffered"] = true;

433 |     d["current_a"] = serialized(String(r.i, 3));

434 |     d["power_w"] = serialized(String(r.p, 3));

435 |     d["energy_today_wh"] = serialized(String(r.e, 4));

436 |     d["fan"] = (bool)(r.flags & 1); d["led"] = (bool)(r.flags & 2);

437 |     d["auto_mode"] = (bool)(r.flags & 4); d["safe_mode"] = (bool)(r.flags & 8);

438 |     d["peak_limit_w"] = peak_limit_w;

439 |     publishJson(T_TELEMETRY, d);

440 |     buf_count--;

441 |   }

442 | }

443 |

444 | void publishTelemetry() {

445 |   seq++;

446 |   StaticJsonDocument<768> d;

447 |   d["dev"] = DEVICE_ID; d["ts"] = epochMs(); d["seq"] = seq;

448 |   d["current_a"] = serialized(String(current_a, 3));

449 |   d["power_w"] = serialized(String(demand_w, 3));

450 |   d["energy_today_wh"] = serialized(String(energy_today_wh, 4));

451 |   d["energy_total_wh"] = serialized(String(energy_total_wh, 3));

452 |   d["peak_today_w"] = serialized(String(peak_today_w, 3));

453 |   d["fan"] = fan_on; d["led"] = led_on; d["led_shed"] = shedName(led_shed);

454 |   d["auto_mode"] = auto_mode; d["safe_mode"] = safe_mode;

455 |   d["peak_limit_w"] = peak_limit_w; d["hysteresis_w"] = hysteresis_w; d["budget_wh"] = budget_wh;

456 |   d["led_est_w"] = serialized(String(led_est_w, 2));

457 |   d["alarm_over"] = alarm_over; d["alarm_budget"] = alarm_budget;

458 |   d["sensor_mv"] = serialized(String(last_mv, 1)); d["zero_mv"] = serialized(String(zero_mv, 1));

459 |   d["diff_mv"] = serialized(String(diff_mv, 1));

460 |   d["rssi"] = WiFi.RSSI(); d["uptime_s"] = millis() / 1000;

461 |   d["time_src"] = time_src; d["rtc"] = have_rtc; d["buffered_count"] = buf_count;

462 |   bool sent = mqtt.connected();

463 |   publishJson(T_TELEMETRY, d);

464 |

465 |   char t[16]; clockStr(t, sizeof(t));

466 |   Serial.printf("[#%lu %s] %.1fmV (lech %+.1fmV) I=%.3fA P=%.2fW E=%.3fWh | FAN:%s LED:%s(%s) | %s%s | MQTT:%s buf=%d\n",

467 |     seq, t, last_mv, diff_mv, current_a, demand_w, energy_today_wh, fan_on ? "ON" : "OFF", led_on ? "ON" : "OFF",

468 |     shedName(led_shed), auto_mode ? "AUTO" : "MANUAL", safe_mode ? " SAFE" : "",

469 |     sent ? "DA GUI" : "CHUA GUI", buf_count);

470 | }

471 |

472 | // -------------------------------------------------------------------------------------

473 | // 11. XỬ LÝ LỆNH

474 | // -------------------------------------------------------------------------------------

475 | void onMqtt(char* topic, byte* payload, unsigned int len) {

476 |   StaticJsonDocument<512> in;

477 |   if (deserializeJson(in, (const char*)payload, len)) return;

478 |

479 |   char cmd_id[40] = "";

480 |   strlcpy(cmd_id, in["cmd_id"] | "", sizeof(cmd_id));

481 |   uint64_t t_cmd = in["ts"] | (uint64_t)0;

482 |   bool ok = true; String err = "";

483 |

484 |   if (in["fan"].is<bool>() || in["led"].is<bool>()) {

485 |     if (auto_mode) { auto_mode = false; publishEvent("MANUAL_OVERRIDE", "lenh tay -> chuyen MANUAL"); }

486 |     if (in["fan"].is<bool>()) { fan_on = in["fan"]; setRelay(PIN_RELAY_FAN, fan_on); }

487 |     if (in["led"].is<bool>()) switchLed(in["led"].as<bool>(), SHED_NONE);

488 |   }

489 |   if (in["auto_mode"].is<bool>()) auto_mode = in["auto_mode"];

490 |   if (!in["peak_limit_w"].isNull()) {

491 |     float v = in["peak_limit_w"]; if (v >= 0.5 && v <= 60) peak_limit_w = v; else { ok = false; err += "peak_limit_w ngoai [0.5,60]; "; }

492 |   }

493 |   if (!in["hysteresis_w"].isNull()) {

494 |     float v = in["hysteresis_w"]; if (v >= 0.1 && v <= 10) hysteresis_w = v; else { ok = false; err += "hysteresis_w ngoai [0.1,10]; "; }

495 |   }

496 |   if (!in["budget_wh"].isNull()) {

497 |     float v = in["budget_wh"]; if (v >= 0 && v <= 10000) { budget_wh = v; alarm_budget = false; budget_warned = false; }

498 |     else { ok = false; err += "budget_wh ngoai [0,10000]; "; }

499 |   }

500 |   if (in["reset_energy"] == true) { energy_today_wh = 0; peak_today_w = 0; alarm_budget = false; budget_warned = false; }

501 |   if (in["calibrate"] == true) {

502 |     if (fan_on || led_on) { ok = false; err += "tat ca 2 tai truoc khi calib; "; } else calibrateZero();

503 |   }

504 |

505 |   prefs.putFloat("limit", peak_limit_w); prefs.putFloat("hyst", hysteresis_w);

506 |   prefs.putFloat("budget", budget_wh);   prefs.putBool("auto", auto_mode);

507 |

508 |   StaticJsonDocument<512> ack;

509 |   ack["cmd_id"] = cmd_id; ack["cmd_ts"] = t_cmd; ack["ok"] = ok;

510 |   if (!ok) ack["error"] = err;

511 |   fillState(ack);

512 |   publishJson(T_ACK, ack);

513 |   publishState();

514 |   Serial.printf("[CMD] id=%s ok=%d %s\n", cmd_id, ok, err.c_str());

515 | }

516 |

517 | // -------------------------------------------------------------------------------------

518 | // 12. MẠNG (không chặn)

519 | // -------------------------------------------------------------------------------------

520 | void networkTask() {

521 |   unsigned long now = millis();

522 |

523 |   static unsigned long last_diag = 0;

524 |   if (!mqtt.connected() && now - last_diag > 5000) {          // nhac trang thai moi 5 giay

525 |     last_diag = now;

526 |     if (WiFi.status() != WL_CONNECTED)

527 |       Serial.printf("[CHAN DOAN] WiFi \"%s\": %s (ma %d)\n", WIFI_SSID,

528 |                     wifiErr(WiFi.status()), WiFi.status());

529 |     else

530 |       Serial.printf("[CHAN DOAN] WiFi OK, IP %s, RSSI %d dBm | MQTT chua noi duoc %s:%d\n",

531 |                     WiFi.localIP().toString().c_str(), WiFi.RSSI(), MQTT_BROKER, MQTT_PORT);

532 |   }

533 |

534 |   if (WiFi.status() != WL_CONNECTED) {

535 |     if (now - last_wifi_try > 15000) {

536 |       last_wifi_try = now; WiFi.disconnect(); WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

537 |       Serial.println("[NET] Thu ket noi WiFi lai...");

538 |     }

539 |   } else if (!mqtt.connected()) {

540 |     if (now - last_mqtt_try > 5000) {

541 |       last_mqtt_try = now;

542 |       bool auth = strlen(MQTT_USER) > 0;

543 |       bool c = mqtt.connect(client_id.c_str(), auth ? MQTT_USER : nullptr, auth ? MQTT_PASS : nullptr,

544 |                             T_STATUS, 1, true, "offline");

545 |       if (c) {

546 |         mqtt.publish(T_STATUS, "online", true);

547 |         mqtt.subscribe(T_CMD, 1);

548 |         if (safe_mode) {

549 |           char b[80];

550 |           snprintf(b, sizeof(b), "mat ket noi %lus, gui bu %d goi", (now - mqtt_lost_ms) / 1000, buf_count);

551 |           safe_mode = false; publishEvent("RECOVERED", b);

552 |         }

553 |         mqtt_lost_ms = 0;

554 |         publishState();

555 |         Serial.printf("[NET] >>> MQTT DA KET NOI. Publish len: %s <<<\n", T_TELEMETRY);

556 |       } else {

557 |         Serial.printf("[NET] MQTT loi rc=%d (%s)\n", mqtt.state(), mqttErr(mqtt.state()));

558 |       }

559 |     }

560 |   }

561 |

562 |   if (mqtt.connected()) {

563 |     mqtt.loop();

564 |     if (buf_count && now - last_flush > 200) { last_flush = now; bufferFlush(5); }   // gửi bù từ từ

565 |   } else {

566 |     if (!mqtt_lost_ms) mqtt_lost_ms = now ? now : 1;

567 |     if (!safe_mode && now - mqtt_lost_ms > OFFLINE_SAFE_MS) {

568 |       safe_mode = true;

569 |       Serial.println("[SAFE] Mat broker > 30s -> quan ly tai cuc bo, bat dau dem du lieu");

570 |     }

571 |   }

572 |

573 |   // NTP -> RTC mỗi 10 phút khi online

574 |   static unsigned long last_sync = 0;

575 |   if (WiFi.status() == WL_CONNECTED && epochS() && (!last_sync || now - last_sync > 600000)) {

576 |     last_sync = now; time_src = "ntp"; syncRtcFromNtp();

577 |   }

578 | }

579 |

580 | // -------------------------------------------------------------------------------------

581 | void setup() {

582 |   Serial.begin(115200);

583 |   delay(500);

584 |   pinMode(PIN_RELAY_FAN, OUTPUT); setRelay(PIN_RELAY_FAN, false);

585 |   pinMode(PIN_RELAY_LED, OUTPUT); setRelay(PIN_RELAY_LED, false);

586 |   if (PIN_BUZZER >= 0) { pinMode(PIN_BUZZER, OUTPUT); buzzer(false); }

587 |

588 |   analogReadResolution(12);

589 |   analogSetPinAttenuation(PIN_ACS712, ADC_11db);

590 |

591 | #if USE_RTC || USE_OLED

592 |   Wire.begin(PIN_I2C_SDA, PIN_I2C_SCL);

593 |   Wire.setClock(400000);

594 |   initRTC();

595 |   initOLED();

596 | #endif

597 |

598 |   prefs.begin("energy", false);

599 |   peak_limit_w    = prefs.getFloat("limit", peak_limit_w);

600 |   hysteresis_w    = prefs.getFloat("hyst", hysteresis_w);

601 |   budget_wh       = prefs.getFloat("budget", budget_wh);

602 |   auto_mode       = prefs.getBool("auto", auto_mode);

603 |   energy_today_wh = prefs.getDouble("e_today", 0);

604 |   energy_total_wh = prefs.getDouble("e_total", 0);

605 |   day_key         = prefs.getLong("day", 0);

606 |

607 |   Serial.println("\n==========================================================");

608 |   Serial.println("  SMART ENERGY v21  -  IoT PROJECT 03");

609 |   Serial.printf("  Device ID : %s\n", DEVICE_ID);

610 |   Serial.printf("  Broker    : %s:%d\n", MQTT_BROKER, MQTT_PORT);

611 |   Serial.printf("  Telemetry : %s\n", T_TELEMETRY);

612 |   Serial.printf("  Status    : %s\n", T_STATUS);

613 |   Serial.printf("  Lenh      : %s\n", T_CMD);

614 |   Serial.printf("  WiFi SSID : %s\n", WIFI_SSID);

615 |   Serial.printf("  RTC: %s | OLED: %s\n", USE_RTC ? "BAT" : "TAT", USE_OLED ? "BAT" : "TAT");

616 |   Serial.println("  Topic tren web PHAI trung voi dong Telemetry o tren!");

617 |   Serial.println("==========================================================");

618 |   Serial.println("Giu 2 tai TAT trong 1 giay de calib diem 0...");

619 |   calibrateZero();

620 |

621 |   uint64_t mac = ESP.getEfuseMac();

622 |   client_id = String("SE_") + DEVICE_ID + "_" + String((uint32_t)(mac & 0xFFFFFFFF), HEX);

623 |

624 |   WiFi.mode(WIFI_STA);

625 |   WiFi.setAutoReconnect(true);

626 |   WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

627 |   last_wifi_try = millis();

628 |   configTime(7 * 3600, 0, "pool.ntp.org", "time.google.com");

629 |

630 |   mqtt.setServer(MQTT_BROKER, MQTT_PORT);

631 |   mqtt.setBufferSize(1024);

632 |   mqtt.setKeepAlive(15);

633 |   mqtt.setSocketTimeout(3);

634 |   mqtt.setCallback(onMqtt);

635 | }

636 |

637 | void loop() {

638 |   unsigned long now = millis();

639 |

640 |   if (now - last_block >= BLOCK_MS) measureBlock();

641 |

642 |   if (now - last_tele >= TELEMETRY_MS) {

643 |     last_tele = now;

644 |     demand_w = n_block ? sum_block_p / n_block : 0;

645 |     sum_block_p = 0; n_block = 0;

646 |     controlStep();                       // điều khiển cục bộ trước

647 |     if (mqtt.connected()) publishTelemetry();

648 |     else bufferPush();                   // mất mạng: đệm lại để gửi bù

649 |   }

650 |

651 |   if (now - last_oled >= OLED_MS) { last_oled = now; drawOLED(); }

652 |

653 |   if (now - last_save >= SAVE_MS) {

654 |     last_save = now;

655 |     prefs.putDouble("e_today", energy_today_wh);

656 |     prefs.putDouble("e_total", energy_total_wh);

657 |     prefs.putLong("day", day_key);

658 |   }

659 |

660 |   networkTask();

661 | }

662 |

# APPENDIX B. COMPLETE BACKEND SOURCE

File backend/main.py. Runs on FastAPI with Uvicorn, using paho-mqtt and psycopg. Without the DATABASE_URL variable it uses SQLite; with it, PostgreSQL.

1 | """

2 | Project 03 - Smart Energy Monitoring & Intelligent Load Management

3 | Backend: FastAPI + MQTT ingest + time-series DB (SQLite hoặc PostgreSQL/Neon)

4 |

5 | Chọn CSDL bằng biến môi trường DATABASE_URL:

6 |   - không đặt            -> SQLite file backend/energy_data.db  (chạy trên laptop)

7 |   - postgresql://...     -> PostgreSQL / Neon                   (chạy 24/7 trên Render)

8 |

9 | Chạy:  pip install -r requirements.txt && python main.py   ->  http://localhost:8000

10 | """

11 | import csv

12 | import io

13 | import json

14 | import os

15 | import sqlite3

16 | import threading

17 | import time

18 | import uuid

19 | from contextlib import asynccontextmanager

20 | from datetime import datetime, timedelta, timezone

21 | from typing import Optional

22 |

23 | import paho.mqtt.client as mqtt

24 | import uvicorn

25 | from fastapi import Depends, FastAPI, Header, HTTPException, Query

26 | from fastapi.middleware.cors import CORSMiddleware

27 | from fastapi.responses import FileResponse, StreamingResponse

28 | from pydantic import BaseModel, Field

29 |

30 | # ----------------------------------------------------------------------------------

31 | # Cấu hình

32 | # ----------------------------------------------------------------------------------

33 | BASE_DIR = os.path.dirname(os.path.abspath(__file__))

34 | DATABASE_URL = os.getenv("DATABASE_URL", "")

35 | DB_FILE = os.getenv("DB_FILE", os.path.join(BASE_DIR, "energy_data.db"))

36 | # Dashboard nằm ở gốc repo (để Vercel phục vụ trực tiếp); vẫn hỗ trợ vị trí cũ

37 | DASHBOARD = next((p for p in (

38 |     os.path.join(BASE_DIR, "..", "index.html"),

39 |     os.path.join(BASE_DIR, "..", "dashboard", "index.html"),

40 | ) if os.path.exists(p)), os.path.join(BASE_DIR, "..", "index.html"))

41 |

42 | MQTT_HOST = os.getenv("MQTT_HOST", "broker.hivemq.com")

43 | MQTT_PORT = int(os.getenv("MQTT_PORT", "1883"))

44 | MQTT_USER = os.getenv("MQTT_USER", "")

45 | MQTT_PASS = os.getenv("MQTT_PASS", "")

46 | DEVICE_ID = os.getenv("DEVICE_ID", "node9988")

47 | BASE = f"smartenergy/{DEVICE_ID}"

48 | TOPICS = {k: f"{BASE}/{k}" for k in ("telemetry", "status", "state", "event", "cmd", "ack")}

49 |

50 | API_KEY = os.getenv("API_KEY", "")   # đặt khi deploy công khai: mọi POST/PUT phải có header X-API-Key

51 | TZ = timezone(timedelta(hours=int(os.getenv("TZ_OFFSET_H", "7"))))

52 | RETENTION_DAYS = int(os.getenv("RETENTION_DAYS", "60"))   # dọn dữ liệu cũ (Neon free 0.5 GB)

53 | STALE_S = 6.0

54 | HTTP_PORT = int(os.getenv("PORT", "8000"))

55 |

56 | now_ms = lambda: int(time.time() * 1000)

57 |

58 |

59 | # ----------------------------------------------------------------------------------

60 | # Lớp CSDL dùng chung cho SQLite và PostgreSQL

61 | # ----------------------------------------------------------------------------------

62 | class DB:

63 |     def __init__(self, url: str, sqlite_path: str):

64 |         self.pg = url.startswith(("postgres://", "postgresql://"))

65 |         if self.pg:

66 |             import psycopg

67 |             from psycopg.rows import dict_row

68 |             from psycopg_pool import ConnectionPool

69 |             self.pool = ConnectionPool(url, min_size=1, max_size=5, timeout=15,

70 |                                        kwargs={"row_factory": dict_row, "autocommit": True})

71 |             self.pool.wait(timeout=30)

72 |             print(f"[DB] PostgreSQL: {url.split('@')[-1].split('?')[0]}")

73 |         else:

74 |             self.conn = sqlite3.connect(sqlite_path, check_same_thread=False)

75 |             self.conn.row_factory = sqlite3.Row

76 |             self.lock = threading.Lock()

77 |             print(f"[DB] SQLite: {sqlite_path}")

78 |

79 |     def _sql(self, sql: str) -> str:

80 |         return sql.replace("?", "%s") if self.pg else sql

81 |

82 |     def query(self, sql, args=(), one=False):

83 |         if self.pg:

84 |             with self.pool.connection() as c:

85 |                 rows = c.execute(self._sql(sql), args).fetchall()

86 |         else:

87 |             with self.lock:

88 |                 rows = [dict(r) for r in self.conn.execute(sql, args).fetchall()]

89 |         return (rows[0] if rows else None) if one else rows

90 |

91 |     def execute(self, sql, args=()):

92 |         if self.pg:

93 |             with self.pool.connection() as c:

94 |                 c.execute(self._sql(sql), args)

95 |         else:

96 |             with self.lock:

97 |                 self.conn.execute(sql, args)

98 |                 self.conn.commit()

99 |

100 |     def insert_ignore(self, table, cols, args, key):

101 |         ph = ",".join("?" * len(cols))

102 |         tail = f" ON CONFLICT ({key}) DO NOTHING" if self.pg else ""

103 |         head = "INSERT INTO" if self.pg else "INSERT OR IGNORE INTO"

104 |         self.execute(f"{head} {table}({','.join(cols)}) VALUES({ph}){tail}", args)

105 |

106 |     def upsert_setting(self, key, value):

107 |         if self.pg:

108 |             self.execute("INSERT INTO settings VALUES(?,?) ON CONFLICT (key) DO UPDATE SET value=EXCLUDED.value", (key, value))

109 |         else:

110 |             self.execute("INSERT OR REPLACE INTO settings VALUES(?,?)", (key, value))

111 |

112 |     def init_schema(self):

113 |         T = {"PK": "BIGSERIAL PRIMARY KEY", "INT": "BIGINT", "REAL": "DOUBLE PRECISION"} if self.pg else \

114 |             {"PK": "INTEGER PRIMARY KEY AUTOINCREMENT", "INT": "INTEGER", "REAL": "REAL"}

115 |         stmts = [

116 |             f"""CREATE TABLE IF NOT EXISTS telemetry (

117 |                 id {T['PK']}, dev TEXT, ts_device {T['INT']}, ts_server {T['INT']} NOT NULL, seq {T['INT']},

118 |                 current_a {T['REAL']}, power_w {T['REAL']}, energy_today_wh {T['REAL']}, energy_total_wh {T['REAL']},

119 |                 peak_today_w {T['REAL']}, fan {T['INT']}, led {T['INT']}, led_shed TEXT, auto_mode {T['INT']},

120 |                 safe_mode {T['INT']}, peak_limit_w {T['REAL']}, hysteresis_w {T['REAL']}, budget_wh {T['REAL']},

121 |                 alarm_over {T['INT']}, alarm_budget {T['INT']}, sensor_mv {T['REAL']}, rssi {T['INT']})""",

122 |             "CREATE INDEX IF NOT EXISTS idx_tel_ts ON telemetry(ts_server)",

123 |             f"""CREATE TABLE IF NOT EXISTS events (

124 |                 id {T['PK']}, dev TEXT, ts_device {T['INT']}, ts_server {T['INT']} NOT NULL,

125 |                 type TEXT, detail TEXT, power_w {T['REAL']}, limit_w {T['REAL']})""",

126 |             "CREATE INDEX IF NOT EXISTS idx_evt_ts ON events(ts_server)",

127 |             f"CREATE TABLE IF NOT EXISTS availability (id {T['PK']}, ts_server {T['INT']} NOT NULL, status TEXT)",

128 |             f"""CREATE TABLE IF NOT EXISTS commands (

129 |                 cmd_id TEXT PRIMARY KEY, dev TEXT, source TEXT, payload TEXT, ts_client {T['INT']},

130 |                 ts_seen_server {T['INT']}, ts_ack_server {T['INT']}, ack_ok {T['INT']}, ack_error TEXT,

131 |                 ack_state TEXT, latency_ms {T['INT']})""",

132 |             "CREATE TABLE IF NOT EXISTS settings (key TEXT PRIMARY KEY, value TEXT)",

133 |         ]

134 |         for s in stmts:

135 |             self.execute(s)

136 |         self.insert_ignore("settings", ["key", "value"], ("tariff_vnd_per_kwh", "3000"), "key")

137 |

138 |     def purge(self, days: int):

139 |         cut = now_ms() - days * 86_400_000

140 |         for t in ("telemetry", "events", "availability"):

141 |             self.execute(f"DELETE FROM {t} WHERE ts_server < ?", (cut,))

142 |         self.execute("DELETE FROM commands WHERE COALESCE(ts_seen_server, ts_client) < ?", (cut,))

143 |

144 |

145 | db = DB(DATABASE_URL, DB_FILE)

146 | db.init_schema()

147 | q, exec_ = db.query, db.execute

148 |

149 |

150 | def get_setting(key, default=None):

151 |     r = q("SELECT value FROM settings WHERE key=?", (key,), one=True)

152 |     return r["value"] if r else default

153 |

154 |

155 | # ----------------------------------------------------------------------------------

156 | # MQTT ingest

157 | # ----------------------------------------------------------------------------------

158 | live = {"status": "unknown", "status_ts": 0, "state": None, "mqtt_connected": False}

159 | ack_waiters: dict[str, threading.Event] = {}

160 |

161 | TEL_COLS = ["current_a", "power_w", "energy_today_wh", "energy_total_wh", "peak_today_w",

162 |             "fan", "led", "led_shed", "auto_mode", "safe_mode", "peak_limit_w", "hysteresis_w",

163 |             "budget_wh", "alarm_over", "alarm_budget", "sensor_mv", "rssi"]

164 |

165 |

166 | def on_connect(client, userdata, flags, reason_code, properties):

167 |     live["mqtt_connected"] = not reason_code.is_failure

168 |     print(f"[MQTT] connected to {MQTT_HOST}: {reason_code}")

169 |     for name, t in TOPICS.items():

170 |         client.subscribe(t, qos=0 if name == "telemetry" else 1)

171 |

172 |

173 | def on_disconnect(client, userdata, flags, reason_code, properties):

174 |     live["mqtt_connected"] = False

175 |     print(f"[MQTT] disconnected: {reason_code}")

176 |

177 |

178 | def on_message(client, userdata, msg):

179 |     ts = now_ms()

180 |     kind = msg.topic.rsplit("/", 1)[-1]

181 |     raw = msg.payload.decode("utf-8", "replace")

182 |     try:

183 |         if kind == "status":

184 |             status = raw.strip().lower()

185 |             if status != live["status"]:

186 |                 exec_("INSERT INTO availability(ts_server,status) VALUES(?,?)", (ts, status))

187 |             live.update(status=status, status_ts=ts)

188 |             return

189 |         data = json.loads(raw)

190 |         if kind == "telemetry":

191 |             vals = [int(v) if isinstance(v, bool) else v for v in (data.get(c) for c in TEL_COLS)]

192 |             # Gói gửi bù sau khi mất mạng: lấy mốc thời gian của thiết bị làm ts_server

193 |             ts_row = int(data["ts"]) if data.get("buffered") and data.get("ts") else ts

194 |             exec_(f"INSERT INTO telemetry(dev,ts_device,ts_server,seq,{','.join(TEL_COLS)}) "

195 |                   f"VALUES(?,?,?,?,{','.join('?' * len(TEL_COLS))})",

196 |                   (data.get("dev"), data.get("ts") or None, ts_row, data.get("seq"), *vals))

197 |             if not data.get("buffered") and live["status"] != "online":

198 |                 live.update(status="online", status_ts=ts)

199 |         elif kind == "state":

200 |             live["state"] = data

201 |         elif kind == "event":

202 |             exec_("INSERT INTO events(dev,ts_device,ts_server,type,detail,power_w,limit_w) VALUES(?,?,?,?,?,?,?)",

203 |                   (data.get("dev"), data.get("ts") or None, ts, data.get("type"), data.get("detail"),

204 |                    data.get("power_w"), data.get("limit_w")))

205 |         elif kind == "cmd":

206 |             cidv = data.get("cmd_id") or f"anon-{ts}"

207 |             db.insert_ignore("commands", ["cmd_id", "dev", "source", "payload", "ts_client", "ts_seen_server"],

208 |                              (cidv, DEVICE_ID, data.get("src", "unknown"), raw, data.get("ts"), ts), "cmd_id")

209 |             exec_("UPDATE commands SET ts_seen_server=COALESCE(ts_seen_server,?) WHERE cmd_id=?", (ts, cidv))

210 |         elif kind == "ack":

211 |             cidv = data.get("cmd_id", "")

212 |             row = q("SELECT ts_seen_server FROM commands WHERE cmd_id=?", (cidv,), one=True)

213 |             lat = ts - row["ts_seen_server"] if row and row["ts_seen_server"] else None

214 |             exec_("UPDATE commands SET ts_ack_server=?, ack_ok=?, ack_error=?, ack_state=?, latency_ms=? WHERE cmd_id=?",

215 |                   (ts, int(bool(data.get("ok"))), data.get("error"), raw, lat, cidv))

216 |             live["state"] = {k: v for k, v in data.items() if k not in ("cmd_id", "cmd_ts", "ok", "error")}

217 |             if cidv in ack_waiters:

218 |                 ack_waiters[cidv].set()

219 |     except Exception as e:

220 |         print(f"[MQTT] bad message on {msg.topic}: {e} | {raw[:120]}")

221 |

222 |

223 | mqttc = mqtt.Client(mqtt.CallbackAPIVersion.VERSION2,

224 |                     client_id=f"se_backend_{DEVICE_ID}_{uuid.uuid4().hex[:6]}")

225 | if MQTT_USER:

226 |     mqttc.username_pw_set(MQTT_USER, MQTT_PASS)

227 | mqttc.on_connect, mqttc.on_disconnect, mqttc.on_message = on_connect, on_disconnect, on_message

228 | mqttc.reconnect_delay_set(1, 10)

229 |

230 |

231 | def housekeeping():

232 |     while True:

233 |         time.sleep(6 * 3600)

234 |         try:

235 |             db.purge(RETENTION_DAYS)

236 |             print(f"[DB] purged data older than {RETENTION_DAYS} days")

237 |         except Exception as e:

238 |             print(f"[DB] purge failed: {e}")

239 |

240 |

241 | @asynccontextmanager

242 | async def lifespan(app):

243 |     mqttc.connect_async(MQTT_HOST, MQTT_PORT, keepalive=30)

244 |     mqttc.loop_start()

245 |     threading.Thread(target=housekeeping, daemon=True).start()

246 |     yield

247 |     mqttc.loop_stop()

248 |

249 |

250 | app = FastAPI(title="Smart Energy IoT API", version="18.0", lifespan=lifespan)

251 | app.add_middleware(CORSMiddleware, allow_origins=["*"], allow_methods=["*"], allow_headers=["*"])

252 |

253 |

254 | # ----------------------------------------------------------------------------------

255 | # Tính toán

256 | # ----------------------------------------------------------------------------------

257 | def integrate_wh(rows, max_gap_s=10):

258 |     """Tích phân hình thang P(t) -> Wh; khoảng mất dữ liệu > max_gap_s không tính."""

259 |     wh = 0.0

260 |     for a, b in zip(rows, rows[1:]):

261 |         dt = (b["ts_server"] - a["ts_server"]) / 1000

262 |         if 0 < dt <= max_gap_s and a["power_w"] is not None and b["power_w"] is not None:

263 |             wh += (a["power_w"] + b["power_w"]) / 2 * dt / 3600

264 |     return wh

265 |

266 |

267 | def pct(values, p):

268 |     if not values:

269 |         return None

270 |     s = sorted(values)

271 |     return s[min(len(s) - 1, int(round(p / 100 * (len(s) - 1))))]

272 |

273 |

274 | def day_start_ms(d: datetime):

275 |     return int(d.replace(hour=0, minute=0, second=0, microsecond=0).timestamp() * 1000)

276 |

277 |

278 | def require_key(x_api_key: Optional[str] = Header(None)):

279 |     """Chỉ bật khi biến môi trường API_KEY được đặt (bắt buộc khi backend chạy public)."""

280 |     if API_KEY and x_api_key != API_KEY:

281 |         raise HTTPException(401, "Thiếu hoặc sai X-API-Key")

282 |

283 |

284 | # ----------------------------------------------------------------------------------

285 | # REST API

286 | # ----------------------------------------------------------------------------------

287 | @app.get("/", include_in_schema=False)

288 | def dashboard():

289 |     return FileResponse(DASHBOARD)

290 |

291 |

292 | @app.get("/api/health")

293 | def health():

294 |     return {"ok": True, "mqtt_connected": live["mqtt_connected"], "broker": MQTT_HOST,

295 |             "topic_base": BASE, "db": "postgres" if db.pg else "sqlite"}

296 |

297 |

298 | @app.get("/api/status")

299 | def status():

300 |     last = q("SELECT * FROM telemetry ORDER BY ts_server DESC LIMIT 1", one=True)

301 |     age = (now_ms() - last["ts_server"]) / 1000 if last else None

302 |     tariff = float(get_setting("tariff_vnd_per_kwh", 3000))

303 |     today_rows = q("SELECT ts_server, power_w FROM telemetry WHERE ts_server>=? ORDER BY ts_server",

304 |                    (day_start_ms(datetime.now(TZ)),))

305 |     e_today = integrate_wh(today_rows)

306 |     return {

307 |         "device": DEVICE_ID, "availability": live["status"], "availability_since": live["status_ts"],

308 |         "mqtt_connected": live["mqtt_connected"], "data_age_s": age,

309 |         "stale": age is None or age > STALE_S, "latest": last, "confirmed_state": live["state"],

310 |         "energy_today_wh_backend": round(e_today, 4),

311 |         "cost_today_vnd": round(e_today / 1000 * tariff, 1), "tariff_vnd_per_kwh": tariff,

312 |     }

313 |

314 |

315 | @app.get("/api/history")

316 | def history(minutes: int = Query(60, ge=1, le=10080), bucket_s: Optional[int] = Query(None, ge=1)):

317 |     since = now_ms() - minutes * 60_000

318 |     b = bucket_s or max(2, minutes * 60 // 300)

319 |     rows = q(f"""SELECT (ts_server/{b * 1000})*{b * 1000} AS t, AVG(power_w) AS power_w, MAX(power_w) AS power_max_w,

320 |                  AVG(current_a) AS current_a, MAX(energy_today_wh) AS energy_today_wh, AVG(peak_limit_w) AS peak_limit_w,

321 |                  MAX(led) AS led, MAX(fan) AS fan, COUNT(*) AS n

322 |                  FROM telemetry WHERE ts_server>=? GROUP BY 1 ORDER BY 1""", (since,))

323 |     return {"bucket_s": b, "points": rows}

324 |

325 |

326 | @app.get("/api/energy/daily")

327 | def energy_daily(days: int = Query(7, ge=1, le=90)):

328 |     tariff = float(get_setting("tariff_vnd_per_kwh", 3000))

329 |     today = datetime.now(TZ)

330 |     out = []

331 |     for i in range(days - 1, -1, -1):

332 |         d = today - timedelta(days=i)

333 |         s = day_start_ms(d)

334 |         rows = q("SELECT ts_server, power_w FROM telemetry WHERE ts_server>=? AND ts_server<? ORDER BY ts_server",

335 |                  (s, s + 86_400_000))

336 |         wh = integrate_wh(rows)

337 |         out.append({"date": d.strftime("%Y-%m-%d"), "energy_wh": round(wh, 4),

338 |                     "peak_w": max((r["power_w"] or 0 for r in rows), default=0),

339 |                     "cost_vnd": round(wh / 1000 * tariff, 1), "samples": len(rows)})

340 |     return out

341 |

342 |

343 | @app.get("/api/events")

344 | def events(limit: int = Query(50, le=1000)):

345 |     return q("SELECT * FROM events ORDER BY ts_server DESC LIMIT ?", (limit,))

346 |

347 |

348 | @app.get("/api/availability")

349 | def availability(limit: int = Query(50, le=1000)):

350 |     return q("SELECT * FROM availability ORDER BY ts_server DESC LIMIT ?", (limit,))

351 |

352 |

353 | @app.get("/api/commands")

354 | def commands(limit: int = Query(50, le=1000)):

355 |     return q("SELECT * FROM commands ORDER BY COALESCE(ts_seen_server, ts_client) DESC LIMIT ?", (limit,))

356 |

357 |

358 | class Command(BaseModel):

359 |     fan: Optional[bool] = None

360 |     led: Optional[bool] = None

361 |     auto_mode: Optional[bool] = None

362 |     peak_limit_w: Optional[float] = Field(None, ge=0.5, le=60)

363 |     hysteresis_w: Optional[float] = Field(None, ge=0.1, le=10)

364 |     budget_wh: Optional[float] = Field(None, ge=0, le=10000)

365 |     calibrate: Optional[bool] = None

366 |     reset_energy: Optional[bool] = None

367 |     wait_ack_s: float = Field(3.0, ge=0, le=10, description="Chờ ACK tối đa (giây)")

368 |

369 |

370 | @app.post("/api/command", dependencies=[Depends(require_key)])

371 | def send_command(cmd: Command):

372 |     body = cmd.model_dump(exclude_none=True)

373 |     wait = body.pop("wait_ack_s")

374 |     if not body:

375 |         raise HTTPException(400, "Không có trường lệnh nào")

376 |     cid = "rest-" + uuid.uuid4().hex[:10]

377 |     payload = {"cmd_id": cid, "ts": now_ms(), "src": "rest", **body}

378 |     db.insert_ignore("commands", ["cmd_id", "dev", "source", "payload", "ts_client"],

379 |                      (cid, DEVICE_ID, "rest", json.dumps(payload), payload["ts"]), "cmd_id")

380 |     ev = ack_waiters[cid] = threading.Event()

381 |     mqttc.publish(TOPICS["cmd"], json.dumps(payload), qos=1)

382 |     acked = ev.wait(wait) if wait else False

383 |     ack_waiters.pop(cid, None)

384 |     return {"cmd_id": cid, "requested": body, "confirmed": acked,

385 |             "command": q("SELECT * FROM commands WHERE cmd_id=?", (cid,), one=True)}

386 |

387 |

388 | @app.post("/api/config", dependencies=[Depends(require_key)])

389 | def set_config(cmd: Command):

390 |     """Alias của /api/command, chỉ nhận các trường cấu hình."""

391 |     if cmd.fan is not None or cmd.led is not None:

392 |         raise HTTPException(400, "Dùng /api/command để bật/tắt tải")

393 |     return send_command(cmd)

394 |

395 |

396 | class Settings(BaseModel):

397 |     tariff_vnd_per_kwh: float = Field(..., gt=0, le=100000)

398 |

399 |

400 | @app.get("/api/settings")

401 | def get_settings():

402 |     return {r["key"]: float(r["value"]) for r in q("SELECT * FROM settings")}

403 |

404 |

405 | @app.put("/api/settings", dependencies=[Depends(require_key)])

406 | def put_settings(s: Settings):

407 |     db.upsert_setting("tariff_vnd_per_kwh", str(s.tariff_vnd_per_kwh))

408 |     return get_settings()

409 |

410 |

411 | @app.get("/api/stats")

412 | def stats(from_ms: Optional[int] = None, to_ms: Optional[int] = None, minutes: int = 10):

413 |     """Số liệu cho phần Thực nghiệm: managed vs unmanaged, độ trễ, tỉ lệ nhận gói."""

414 |     to_ms = to_ms or now_ms()

415 |     from_ms = from_ms or to_ms - minutes * 60_000

416 |     rows = q("SELECT * FROM telemetry WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server", (from_ms, to_ms))

417 |     ev = q("SELECT type, COUNT(*) AS n FROM events WHERE ts_server BETWEEN ? AND ? GROUP BY type", (from_ms, to_ms))

418 |     cmds = q("SELECT latency_ms, ack_ok FROM commands WHERE ts_seen_server BETWEEN ? AND ?", (from_ms, to_ms))

419 |

420 |     missing, over_s, led_switches = 0, 0.0, 0

421 |     for a, b in zip(rows, rows[1:]):

422 |         if a["seq"] is not None and b["seq"] is not None and b["seq"] > a["seq"]:

423 |             missing += b["seq"] - a["seq"] - 1

424 |         dt = (b["ts_server"] - a["ts_server"]) / 1000

425 |         if dt <= 10 and a["power_w"] is not None and a["peak_limit_w"] is not None and a["power_w"] > a["peak_limit_w"]:

426 |             over_s += dt

427 |         if a["led"] != b["led"]:

428 |             led_switches += 1

429 |     lat = [r["ts_server"] - r["ts_device"] for r in rows if r["ts_device"]]

430 |     clat = [c["latency_ms"] for c in cmds if c["latency_ms"] is not None]

431 |     powers = [r["power_w"] for r in rows if r["power_w"] is not None]

432 |     n = len(rows)

433 |     return {

434 |         "window": {"from_ms": from_ms, "to_ms": to_ms, "duration_s": (to_ms - from_ms) / 1000},

435 |         "packets": {"received": n, "missing_by_seq": missing,

436 |                     "success_rate": round(n / (n + missing), 4) if n else None},

437 |         "power": {"peak_w": max(powers, default=None), "avg_w": round(sum(powers) / n, 3) if n else None,

438 |                   "energy_wh": round(integrate_wh(rows), 4), "over_limit_duration_s": round(over_s, 1)},

439 |         "control": {"led_switch_count": led_switches, "events": {e["type"]: e["n"] for e in ev}},

440 |         "latency_device_to_backend_ms": {"n": len(lat), "avg": round(sum(lat) / len(lat), 1) if lat else None,

441 |                                          "p95": pct(lat, 95), "max": max(lat, default=None),

442 |                                          "note": "cần NTP đồng bộ trên ESP32 và máy chạy backend"},

443 |         "latency_cmd_to_ack_ms": {"n": len(clat), "avg": round(sum(clat) / len(clat), 1) if clat else None,

444 |                                   "p95": pct(clat, 95), "max": max(clat, default=None),

445 |                                   "unacked": sum(1 for c in cmds if c["ack_ok"] is None)},

446 |     }

447 |

448 |

449 | @app.get("/api/export/{table}.csv")

450 | def export_csv(table: str, minutes: int = Query(1440, ge=1, le=525600)):

451 |     if table not in ("telemetry", "events", "commands", "availability"):

452 |         raise HTTPException(404, "table không hợp lệ")

453 |     col = "ts_seen_server" if table == "commands" else "ts_server"

454 |     rows = q(f"SELECT * FROM {table} WHERE {col}>=? ORDER BY {col}", (now_ms() - minutes * 60_000,))

455 |     buf = io.StringIO()

456 |     if rows:

457 |         w = csv.DictWriter(buf, fieldnames=list(rows[0].keys()))

458 |         w.writeheader()

459 |         w.writerows(rows)

460 |     return StreamingResponse(iter([buf.getvalue()]), media_type="text/csv",

461 |                              headers={"Content-Disposition": f"attachment; filename={table}.csv"})

462 |

463 |

464 |

465 | # ----------------------------------------------------------------------------------

466 | # Chatbot offline: phân loại ý định -> truy vấn SQL -> trả lời từ số liệu thật

467 | # Không dùng LLM nên không bao giờ bịa số; mọi câu trả lời kèm SQL đã chạy.

468 | # ----------------------------------------------------------------------------------

469 | import re

470 | import unicodedata

471 |

472 |

473 | def _norm(t: str) -> str:

474 |     t = unicodedata.normalize("NFD", t.lower().replace("đ", "d").replace("Đ", "d"))

475 |     return "".join(c for c in t if unicodedata.category(c) != "Mn")

476 |

477 |

478 | def _range(t: str):

479 |     now = datetime.now(TZ)

480 |     if re.search(r"hom qua|yesterday", t):

481 |         s = day_start_ms(now - timedelta(days=1))

482 |         return s, s + 86_400_000, "hôm qua"

483 |     if re.search(r"7 ngay|tuan|week", t):

484 |         return day_start_ms(now - timedelta(days=6)), now_ms(), "7 ngày qua"

485 |     if re.search(r"30 ngay|thang|month", t):

486 |         return day_start_ms(now - timedelta(days=29)), now_ms(), "30 ngày qua"

487 |     if re.search(r"gio qua|1 gio|60 phut|hour", t):

488 |         return now_ms() - 3_600_000, now_ms(), "1 giờ qua"

489 |     return day_start_ms(now), now_ms(), "hôm nay"

490 |

491 |

492 | def _rows(a, b):

493 |     return q("SELECT ts_server, power_w, peak_limit_w, led, fan FROM telemetry "

494 |              "WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server", (a, b))

495 |

496 |

497 | def _tariff():

498 |     return float(get_setting("tariff_vnd_per_kwh", 3000))

499 |

500 |

501 | def chat_answer(question: str) -> dict:

502 |     t = _norm(question.strip())

503 |     a, b, label = _range(t)

504 |     tariff = _tariff()

505 |

506 |     # --- 1. Lệnh điều khiển: trả về action để người dùng bấm xác nhận ---

507 |     on = bool(re.search(r"\bbat\b|\bmo\b|turn on|\bon\b", t))

508 |     off = bool(re.search(r"\btat\b|\bngat\b|turn off|\boff\b", t))

509 |     if on or off:

510 |         if re.search(r"quat|fan", t):

511 |             return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} quạt 12V? Bấm xác nhận để gửi lệnh.",

512 |                     "action": {"fan": on}, "data": {}}

513 |         if re.search(r"den|led", t):

514 |             return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} đèn LED? Bấm xác nhận để gửi lệnh.",

515 |                     "action": {"led": on}, "data": {}}

516 |         if re.search(r"tu dong|auto", t):

517 |             return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} chế độ tự động sa thải tải?",

518 |                     "action": {"auto_mode": on}, "data": {}}

519 |     m = re.search(r"(nguong|limit|peak)\D{0,12}(\d+(?:[.,]\d+)?)", t)

520 |     if m:

521 |         v = float(m.group(2).replace(",", "."))

522 |         return {"answer": f"Đặt ngưỡng công suất đỉnh thành {v} W?", "action": {"peak_limit_w": v}, "data": {}}

523 |     m = re.search(r"(ngan sach|budget)\D{0,12}(\d+(?:[.,]\d+)?)", t)

524 |     if m:

525 |         v = float(m.group(2).replace(",", "."))

526 |         return {"answer": f"Đặt ngân sách điện năng {v} Wh/ngày?", "action": {"budget_wh": v}, "data": {}}

527 |

528 |     # --- 2. Trạng thái hiện tại ---

529 |     if re.search(r"hien tai|bay gio|dang chay|status|trang thai|the nao", t):

530 |         st = status()

531 |         l = st["latest"]

532 |         if not l:

533 |             return {"answer": "Chưa có dữ liệu nào trong database.", "data": {}, "action": None}

534 |         return {"answer": (f"Thiết bị {st['availability'].upper()}, dữ liệu cách đây {st['data_age_s']:.0f} s. "

535 |                            f"Công suất {l['power_w']:.2f} W / ngưỡng {l['peak_limit_w']:.1f} W, "

536 |                            f"dòng {l['current_a']:.3f} A. Quạt {'BẬT' if l['fan'] else 'TẮT'}, "

537 |                            f"đèn {'BẬT' if l['led'] else 'TẮT'}"

538 |                            + (f" (bị cắt do {l['led_shed']})" if l['led_shed'] not in (None, 'NONE') else "")

539 |                            + f". Chế độ {'AUTO' if l['auto_mode'] else 'MANUAL'}"

540 |                            + (" (SAFE MODE)" if l['safe_mode'] else "") + "."),

541 |                 "data": l, "action": None}

542 |

543 |     # --- 3. Đỉnh công suất ---

544 |     if re.search(r"dinh|peak|cao nhat|max", t):

545 |         rows = _rows(a, b)

546 |         if not rows:

547 |             return {"answer": f"Không có dữ liệu {label}.", "data": {}, "action": None}

548 |         top = max(rows, key=lambda r: r["power_w"] or 0)

549 |         return {"answer": (f"Đỉnh công suất {label} là {top['power_w']:.2f} W lúc "

550 |                            f"{datetime.fromtimestamp(top['ts_server'] / 1000, TZ):%H:%M:%S %d/%m}."),

551 |                 "data": {"peak_w": top["power_w"], "ts": top["ts_server"]}, "action": None}

552 |

553 |     # --- 4. Sa thải tải / sự kiện ---

554 |     if re.search(r"sa thai|cat tai|shed|su kien|event|canh bao|alarm|bao nhieu lan", t):

555 |         ev = q("SELECT type, COUNT(*) AS n FROM events WHERE ts_server BETWEEN ? AND ? GROUP BY type ORDER BY 2 DESC", (a, b))

556 |         if not ev:

557 |             return {"answer": f"Không có sự kiện nào {label}.", "data": {}, "action": None}

558 |         last = q("SELECT * FROM events WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server DESC LIMIT 1", (a, b), one=True)

559 |         txt = ", ".join(f"{e['type']}: {e['n']} lần" for e in ev)

560 |         return {"answer": (f"{label.capitalize()} có {sum(e['n'] for e in ev)} sự kiện — {txt}. "

561 |                            f"Gần nhất: {last['type']} lúc {datetime.fromtimestamp(last['ts_server'] / 1000, TZ):%H:%M:%S} "

562 |                            f"({last['detail']}, P={last['power_w']:.2f} W)."),

563 |                 "data": {"by_type": ev}, "action": None}

564 |

565 |     # --- 5. Mất kết nối ---

566 |     if re.search(r"offline|mat ket noi|mat mang|online|ket noi", t):

567 |         av = q("SELECT * FROM availability WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server DESC LIMIT 10", (a, b))

568 |         n_off = sum(1 for r in av if r["status"] == "offline")

569 |         if not av:

570 |             return {"answer": f"Không ghi nhận thay đổi kết nối nào {label}.", "data": {}, "action": None}

571 |         return {"answer": (f"{label.capitalize()} có {n_off} lần thiết bị OFFLINE. Gần nhất: "

572 |                            f"{av[0]['status'].upper()} lúc {datetime.fromtimestamp(av[0]['ts_server'] / 1000, TZ):%H:%M:%S %d/%m}."),

573 |                 "data": {"log": av}, "action": None}

574 |

575 |     # --- 6. Lệnh điều khiển đã gửi ---

576 |     if re.search(r"lenh|command|dieu khien gan day|ai bat|ai tat", t):

577 |         c = q("SELECT * FROM commands WHERE COALESCE(ts_seen_server, ts_client) BETWEEN ? AND ? "

578 |               "ORDER BY COALESCE(ts_seen_server, ts_client) DESC LIMIT 5", (a, b))

579 |         if not c:

580 |             return {"answer": f"Không có lệnh nào {label}.", "data": {}, "action": None}

581 |         lat = [x["latency_ms"] for x in c if x["latency_ms"] is not None]

582 |         lines = "; ".join(f"{datetime.fromtimestamp((x['ts_seen_server'] or x['ts_client']) / 1000, TZ):%H:%M:%S} "

583 |                           f"{json.loads(x['payload']).get('src', '?')}: "

584 |                           f"{ {k: v for k, v in json.loads(x['payload']).items() if k not in ('cmd_id', 'ts', 'src')} }"

585 |                           for x in c)

586 |         return {"answer": (f"{len(c)} lệnh gần nhất {label}: {lines}."

587 |                            + (f" Độ trễ xác nhận trung bình {sum(lat) / len(lat):.0f} ms." if lat else "")),

588 |                 "data": {"commands": c}, "action": None}

589 |

590 |     # --- 7. So sánh theo ngày ---

591 |     if re.search(r"moi ngay|theo ngay|so sanh|bieu do ngay", t):

592 |         d = energy_daily(7)

593 |         txt = ", ".join(f"{x['date'][5:]}: {x['energy_wh']:.1f} Wh" for x in d)

594 |         return {"answer": f"Điện năng 7 ngày gần nhất — {txt}.", "data": {"daily": d}, "action": None}

595 |

596 |     # --- 8. Điện năng / tiền điện (mặc định) ---

597 |     if not re.search(r"dien|wh|kwh|tien|chi phi|cost|tieu thu|ton|nang luong|energy|cong suat|power|bao nhieu", t):

598 |         return {"answer": ("Mình tra cứu được dữ liệu trong database. Thử hỏi: điện năng/tiền điện theo ngày, "

599 |                            "đỉnh công suất, số lần sa thải tải, thiết bị offline lúc nào, lệnh điều khiển gần đây, "

600 |                            "trạng thái hiện tại. Hoặc ra lệnh: 'bật quạt', 'tắt đèn', 'đặt ngưỡng 3.5 W'."),

601 |                 "data": {}, "action": None}

602 |     rows = _rows(a, b)

603 |     if True:

604 |         if not rows:

605 |             return {"answer": f"Chưa có dữ liệu {label}. Backend chỉ ghi được khi ESP32 đang gửi telemetry.",

606 |                     "data": {}, "action": None}

607 |         wh = integrate_wh(rows)

608 |         peak = max((r["power_w"] or 0) for r in rows)

609 |         avg = sum(r["power_w"] or 0 for r in rows) / len(rows)

610 |         hours = (rows[-1]["ts_server"] - rows[0]["ts_server"]) / 3_600_000

611 |         return {"answer": (f"{label.capitalize()}: tiêu thụ {wh:.3f} Wh ({wh / 1000:.5f} kWh) "

612 |                            f"≈ {wh / 1000 * tariff:,.0f} đ với đơn giá {tariff:,.0f} đ/kWh. "

613 |                            f"Công suất trung bình {avg:.2f} W, đỉnh {peak:.2f} W, "

614 |                            f"đo trong {hours:.1f} giờ ({len(rows)} mẫu)."),

615 |                 "data": {"energy_wh": round(wh, 4), "cost_vnd": round(wh / 1000 * tariff, 1),

616 |                          "peak_w": peak, "avg_w": round(avg, 3), "samples": len(rows)},

617 |                 "action": None}

618 |

619 |

620 | class ChatIn(BaseModel):

621 |     question: str = Field(..., min_length=1, max_length=300)

622 |

623 |

624 | @app.post("/api/chat")

625 | def chat(inp: ChatIn):

626 |     """Chatbot tra cứu dữ liệu. Lệnh điều khiển trả về 'action' để người dùng bấm xác nhận

627 |     (dashboard sẽ gọi /api/command), chatbot không tự đóng cắt relay."""

628 |     try:

629 |         r = chat_answer(inp.question)

630 |     except Exception as e:

631 |         raise HTTPException(500, f"Lỗi xử lý câu hỏi: {e}")

632 |     r["question"] = inp.question

633 |     r["hint"] = ("Thử: 'hôm nay dùng bao nhiêu điện', 'tiền điện 7 ngày qua', 'đỉnh công suất hôm nay', "

634 |                  "'sa thải mấy lần', 'thiết bị offline lúc nào', 'lệnh gần đây', 'bật quạt', 'đặt ngưỡng 3.5 W'")

635 |     return r

636 |

637 |

638 | if __name__ == "__main__":

639 |     print(f"Broker: {MQTT_HOST}:{MQTT_PORT}  topics: {BASE}/#\nDashboard: http://localhost:{HTTP_PORT}")

640 |     uvicorn.run(app, host="0.0.0.0", port=HTTP_PORT)

641 |

# APPENDIX C. DATA ASSISTANT SOURCE

Extracted from backend/main.py: intent classification and answer generation from database queries.

def chat_answer(question: str) -> dict:

t = _norm(question.strip())

a, b, label = _range(t)

tariff = _tariff()

# --- 1. Lệnh điều khiển: trả về action để người dùng bấm xác nhận ---

on = bool(re.search(r"\bbat\b|\bmo\b|turn on|\bon\b", t))

off = bool(re.search(r"\btat\b|\bngat\b|turn off|\boff\b", t))

if on or off:

if re.search(r"quat|fan", t):

return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} quạt 12V? Bấm xác nhận để gửi lệnh.",

"action": {"fan": on}, "data": {}}

if re.search(r"den|led", t):

return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} đèn LED? Bấm xác nhận để gửi lệnh.",

"action": {"led": on}, "data": {}}

if re.search(r"tu dong|auto", t):

return {"answer": f"Bạn muốn {'BẬT' if on else 'TẮT'} chế độ tự động sa thải tải?",

"action": {"auto_mode": on}, "data": {}}

m = re.search(r"(nguong|limit|peak)\D{0,12}(\d+(?:[.,]\d+)?)", t)

if m:

v = float(m.group(2).replace(",", "."))

return {"answer": f"Đặt ngưỡng công suất đỉnh thành {v} W?", "action": {"peak_limit_w": v}, "data": {}}

m = re.search(r"(ngan sach|budget)\D{0,12}(\d+(?:[.,]\d+)?)", t)

if m:

v = float(m.group(2).replace(",", "."))

return {"answer": f"Đặt ngân sách điện năng {v} Wh/ngày?", "action": {"budget_wh": v}, "data": {}}

# --- 2. Trạng thái hiện tại ---

if re.search(r"hien tai|bay gio|dang chay|status|trang thai|the nao", t):

st = status()

l = st["latest"]

if not l:

return {"answer": "Chưa có dữ liệu nào trong database.", "data": {}, "action": None}

return {"answer": (f"Thiết bị {st['availability'].upper()}, dữ liệu cách đây {st['data_age_s']:.0f} s. "

f"Công suất {l['power_w']:.2f} W / ngưỡng {l['peak_limit_w']:.1f} W, "

f"dòng {l['current_a']:.3f} A. Quạt {'BẬT' if l['fan'] else 'TẮT'}, "

f"đèn {'BẬT' if l['led'] else 'TẮT'}"

+ (f" (bị cắt do {l['led_shed']})" if l['led_shed'] not in (None, 'NONE') else "")

+ f". Chế độ {'AUTO' if l['auto_mode'] else 'MANUAL'}"

+ (" (SAFE MODE)" if l['safe_mode'] else "") + "."),

"data": l, "action": None}

# --- 3. Đỉnh công suất ---

if re.search(r"dinh|peak|cao nhat|max", t):

rows = _rows(a, b)

if not rows:

return {"answer": f"Không có dữ liệu {label}.", "data": {}, "action": None}

top = max(rows, key=lambda r: r["power_w"] or 0)

return {"answer": (f"Đỉnh công suất {label} là {top['power_w']:.2f} W lúc "

f"{datetime.fromtimestamp(top['ts_server'] / 1000, TZ):%H:%M:%S %d/%m}."),

"data": {"peak_w": top["power_w"], "ts": top["ts_server"]}, "action": None}

# --- 4. Sa thải tải / sự kiện ---

if re.search(r"sa thai|cat tai|shed|su kien|event|canh bao|alarm|bao nhieu lan", t):

ev = q("SELECT type, COUNT(*) AS n FROM events WHERE ts_server BETWEEN ? AND ? GROUP BY type ORDER BY 2 DESC", (a, b))

if not ev:

return {"answer": f"Không có sự kiện nào {label}.", "data": {}, "action": None}

last = q("SELECT * FROM events WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server DESC LIMIT 1", (a, b), one=True)

txt = ", ".join(f"{e['type']}: {e['n']} lần" for e in ev)

return {"answer": (f"{label.capitalize()} có {sum(e['n'] for e in ev)} sự kiện — {txt}. "

f"Gần nhất: {last['type']} lúc {datetime.fromtimestamp(last['ts_server'] / 1000, TZ):%H:%M:%S} "

f"({last['detail']}, P={last['power_w']:.2f} W)."),

"data": {"by_type": ev}, "action": None}

# --- 5. Mất kết nối ---

if re.search(r"offline|mat ket noi|mat mang|online|ket noi", t):

av = q("SELECT * FROM availability WHERE ts_server BETWEEN ? AND ? ORDER BY ts_server DESC LIMIT 10", (a, b))

n_off = sum(1 for r in av if r["status"] == "offline")

if not av:

return {"answer": f"Không ghi nhận thay đổi kết nối nào {label}.", "data": {}, "action": None}

return {"answer": (f"{label.capitalize()} có {n_off} lần thiết bị OFFLINE. Gần nhất: "

f"{av[0]['status'].upper()} lúc {datetime.fromtimestamp(av[0]['ts_server'] / 1000, TZ):%H:%M:%S %d/%m}."),

"data": {"log": av}, "action": None}

# --- 6. Lệnh điều khiển đã gửi ---

if re.search(r"lenh|command|dieu khien gan day|ai bat|ai tat", t):

c = q("SELECT * FROM commands WHERE COALESCE(ts_seen_server, ts_client) BETWEEN ? AND ? "

"ORDER BY COALESCE(ts_seen_server, ts_client) DESC LIMIT 5", (a, b))

if not c:

return {"answer": f"Không có lệnh nào {label}.", "data": {}, "action": None}

lat = [x["latency_ms"] for x in c if x["latency_ms"] is not None]

lines = "; ".join(f"{datetime.fromtimestamp((x['ts_seen_server'] or x['ts_client']) / 1000, TZ):%H:%M:%S} "

f"{json.loads(x['payload']).get('src', '?')}: "

f"{ {k: v for k, v in json.loads(x['payload']).items() if k not in ('cmd_id', 'ts', 'src')} }"

for x in c)

return {"answer": (f"{len(c)} lệnh gần nhất {label}: {lines}."

+ (f" Độ trễ xác nhận trung bình {sum(lat) / len(lat):.0f} ms." if lat else "")),

"data": {"commands": c}, "action": None}

# --- 7. So sánh theo ngày ---

if re.search(r"moi ngay|theo ngay|so sanh|bieu do ngay", t):

d = energy_daily(7)

txt = ", ".join(f"{x['date'][5:]}: {x['energy_wh']:.1f} Wh" for x in d)

return {"answer": f"Điện năng 7 ngày gần nhất — {txt}.", "data": {"daily": d}, "action": None}

# --- 8. Điện năng / tiền điện (mặc định) ---

if not re.search(r"dien|wh|kwh|tien|chi phi|cost|tieu thu|ton|nang luong|energy|cong suat|power|bao nhieu", t):

return {"answer": ("Mình tra cứu được dữ liệu trong database. Thử hỏi: điện năng/tiền điện theo ngày, "

"đỉnh công suất, số lần sa thải tải, thiết bị offline lúc nào, lệnh điều khiển gần đây, "

"trạng thái hiện tại. Hoặc ra lệnh: 'bật quạt', 'tắt đèn', 'đặt ngưỡng 3.5 W'."),

"data": {}, "action": None}

rows = _rows(a, b)

if True:

if not rows:

return {"answer": f"Chưa có dữ liệu {label}. Backend chỉ ghi được khi ESP32 đang gửi telemetry.",

"data": {}, "action": None}

wh = integrate_wh(rows)

peak = max((r["power_w"] or 0) for r in rows)

avg = sum(r["power_w"] or 0 for r in rows) / len(rows)

hours = (rows[-1]["ts_server"] - rows[0]["ts_server"]) / 3_600_000

return {"answer": (f"{label.capitalize()}: tiêu thụ {wh:.3f} Wh ({wh / 1000:.5f} kWh) "

f"≈ {wh / 1000 * tariff:,.0f} đ với đơn giá {tariff:,.0f} đ/kWh. "

f"Công suất trung bình {avg:.2f} W, đỉnh {peak:.2f} W, "

f"đo trong {hours:.1f} giờ ({len(rows)} mẫu)."),

"data": {"energy_wh": round(wh, 4), "cost_vnd": round(wh / 1000 * tariff, 1),

"peak_w": peak, "avg_w": round(avg, 3), "samples": len(rows)},

"action": None}

class ChatIn(BaseModel):

# APPENDIX D. DASHBOARD SOURCE (EXTRACT)

Extracted from index.html: telemetry handling, command dispatch and acknowledgement matching.

function onTelemetry(d) {

const now = Date.now();

S.tele = { ...S.tele, ...d }; S.lastRx = now;

if (S.devStatus !== 'online') onStatus('online');

// thống kê độ trễ & mất gói

if (d.ts > 1.7e12) { S.telLat.push(now - d.ts); if (S.telLat.length > 200) S.telLat.shift(); }

if (typeof d.seq === 'number') {

if (S.lastSeq !== null && d.seq > S.lastSeq) S.missing += d.seq - S.lastSeq - 1;

S.lastSeq = d.seq; S.rxCount++;

}

$('k-power').textContent = fmt(d.power_w);

$('k-limit').textContent = fmt(d.peak_limit_w, 1);

$('k-current').textContent = fmt(d.current_a, 3);

$('k-mv').textContent = `${fmt(d.sensor_mv, 0)} (0 = ${fmt(d.zero_mv, 0)}` +

(d.diff_mv !== undefined ? `, lệch ${d.diff_mv > 0 ? '+' : ''}${fmt(d.diff_mv, 0)}` : '') + ')';

$('k-energy').textContent = fmt(d.energy_today_wh, 3);

$('k-cost').textContent = fmt(d.energy_today_wh / 1000 * S.tariff, 1);

$('k-peak').textContent = fmt(d.peak_today_w);

$('k-total').textContent = fmt(d.energy_total_wh, 2);

$('k-seq').textContent = d.seq; $('k-rssi').textContent = d.rssi;

$('k-ledest').textContent = fmt(d.led_est_w, 2);

if (d.time_src) $('k-timesrc').textContent = d.time_src === 'ntp' ? 'NTP' : d.time_src === 'rtc' ? 'RTC (DS3231)' : 'chưa đồng bộ';

if (d.buffered_count !== undefined) $('k-buf').textContent = d.buffered_count;

$('k-shed').textContent = d.led_shed === 'NONE' ? 'không bị cắt' : 'bị cắt do ' + d.led_shed;

const bud = d.budget_wh > 0;

$('k-budget').textContent = bud ? `${fmt(d.energy_today_wh / d.budget_wh * 100, 0)}% của ${d.budget_wh} Wh` : 'tắt';

const bar = $('k-budget-bar');

bar.style.width = bud ? Math.min(100, d.energy_today_wh / d.budget_wh * 100) + '%' : '0';

bar.className = 'h-1.5 rounded ' + (!bud ? 'bg-slate-600' : d.energy_today_wh >= d.budget_wh ? 'bg-rose-500' : d.energy_today_wh >= .8 * d.budget_wh ? 'bg-amber-500' : 'bg-emerald-500');

onState(d);

rtPush(d.ts > 1.7e12 ? d.ts : now, d.power_w, d.peak_limit_w, d.current_a);

renderAlarms(); renderLoads(); renderExp();

log('TEL', `#${d.seq} I=${fmt(d.current_a, 3)}A P=${fmt(d.power_w)}W E=${fmt(d.energy_today_wh, 3)}Wh FAN=${d.fan ? 'ON' : 'OFF'} LED=${d.led ? 'ON' : 'OFF'} ${d.auto_mode ? 'AUTO' : 'MANUAL'}${d.safe_mode ? ' SAFE' : ''}`, 'text-cyan-300');

}

let configTouched = false;

function publishCmd(body, onDone) {

if (!client.connected) { log('CMD', 'MQTT chưa kết nối, không gửi được', 'text-rose-400'); return null; }

const id = cid();

const payload = { cmd_id: id, ts: Date.now(), src: 'web', ...body };

const t0 = performance.now();

const timer = setTimeout(() => {

delete S.pending[id]; onDone && onDone({ status: 'timeout' }); renderExp();

log('CMD', `Lệnh ${id} không được xác nhận sau ${ACK_TIMEOUT_MS} ms`, 'text-rose-400');

}, ACK_TIMEOUT_MS);

S.pending[id] = { t0, timer, onDone };

client.publish(`${TOPIC}/cmd`, JSON.stringify(payload), { qos: 1 });

log('CMD', `→ ${JSON.stringify(payload)}`, 'text-amber-300');

if (S.devStatus !== 'online') log('CMD', 'Cảnh báo: thiết bị đang OFFLINE, lệnh sẽ không được xác nhận', 'text-amber-400');

renderExp();

return id;

}

function onAck(d) {

const p = S.pending[d.cmd_id];

if (!p) return onState(d);                         // ACK của client khác (backend/REST)

clearTimeout(p.timer); delete S.pending[d.cmd_id];

const latency = Math.round(performance.now() - p.t0);

S.cmdLat.push(latency); if (S.cmdLat.length > 100) S.cmdLat.shift();

onState(d);

p.onDone && p.onDone({ status: d.ok ? 'ok' : 'error', latency, error: d.error });

renderExp();

log('ACK', `← ${d.cmd_id} ok=${d.ok} sau ${latency} ms${d.error ? ' · ' + d.error : ''}`, d.ok ? 'text-emerald-300' : 'text-rose-400');

}

window.sendLoad = (key, value) => {

# APPENDIX E. EXPERIMENT DATA SHEETS

Print these sheets to record readings on the bench, then transfer them into chapter 11.

## E1. Current measurement error

| Run | No load (A) | Fan (A) | Lamp (A) | Both (A) | Reference (A) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| Mean |  |  |  |  |  |
| Std. dev. |  |  |  |  |  |

## E2, E3. Latency

| Run | Device → dashboard (ms) | Command → ack (ms) | Notes |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| Mean |  |  |  |
| 95th percentile |  |  |  |

## E4. Managed versus unmanaged operation

| Metric over 10 minutes | Unmanaged | Managed | Difference (%) |
| --- | --- | --- | --- |
| Peak power (W) |  |  |  |
| Energy (Wh) |  |  |  |
| Time above limit (s) |  |  |  |
| Switching operations |  |  |  |
| Packet success rate (%) |  |  |  |

## E6. Network loss and recovery

| Observation | Result |
| --- | --- |
| Time of disconnection |  |
| Time the badge turned OFFLINE |  |
| Local shedding behaviour during the outage |  |
| Time of reconnection |  |
| Records forwarded |  |
| Residual gap on the chart (s) |  |